Bonneville Power Administration

2001 Action Plan For Fish in Response to Power System Emergency Solicitation of Proposals

# PART 2 of 2. Narrative

Title?:	Lake Roosevelt/Colville Tribes Emergency Fish Restoration

Section 3. Project description

Provide project detail for headings a through g.

a. Abstract?

In the early 1980's, the Confederated Tribes of the Colville Reservation, the Spokane Tribe of Indians and the Washington Department of Fish and Wildlife developed a management strategy for Lake Roosevelt on the restoration and enhancement of Kokanee salmon (Oncorhynchus nerka) and rainbow trout (Oncorhynchus mykiss) populations using large scale hatchery out planting and restoring natural spawning runs in tributaries.  The plan was incorporated by the Northwest Power Planning Council (NPPC) into their 1987 Columbia Basin Fish and Wildlife program as a mitigation measure for hydropower-caused fish losses resulting from the construction of Grand Coulee Dam. Lakes Roosevelt and Rufus Woods are part of the blocked area.

The drought of 2001 resulted in unusual system operations at Grand Coulee Dam. Emergency power generation has potentially resulted in increased entrainment at Grand Coulee Dam, lowered and altered the primary food chain (phyto and zooplankton), and altered the normal refill curve. The recent operation of the hydropower system is very similar to operations common to high water years such as during 1996 and 1997.

Area managers agree that 2001 Grand Coulee operations probably resulted in increased entrainment. Data collected during an earlier hydro acoustic study indicated that entrainment spikes followed net pen releases. A time delay was evident to allow for the distance traveled to the dam (LeCaire, 1999) (Figure 9). Data analysis performed by BioSonics, Inc. indicated that the target strength information correlated to the approximate sizes of the released net pen fish (Sullivan, 1999) (Table 18).

We are proposing to address the lost fish problem by replacing them on a fish for fish basis. We propose to purchase a complement of locally produced legal size, triploid steel head and plant them directly into Lake Roosevelt and purchase a second group to seed into the current Bonneville Power funded net pen operation.  Since the current net pen operation is fully stocked we propose the purchase of additional net pen arrays that offer increased longevity, safety and ultimately increased production.

Concurrent with the net pen operation and fish-stocking proposal we propose the purchase of 20,000 floy tags. This will allow us to determine the success/failure of the triploid rainbow trout to the Lake Roosevelt fishery. This is to replace the group of freshly tagged rainbow released early (May 6, 2001) that may have entrained out of the reservoir. The initial cost outlay will be minimal since only a small number of tags need to be purchased and just a single week of labor by six individuals is necessary to get the study off the ground. Data collection is part of the current lake Roosevelt Monitoring program and the resource managers each have a census taker in place that can collect this additional data.

b. Justification as 2001 Action Plan For Fish in Response to Power System Emergency Solicitation of Proposals

The species at risk is the rainbow trout (Oncorhynchus mykiss). Rainbow trout are a major component of the Lake Franklin D. Roosevelt fishery. Lake Roosevelt is a very popular destination for vacationers, tourists, anglers, and boaters. Revenue generated by recreational and subsistence fishers is estimated in the millions of dollars annually (Scholz et al. 1990). Anglers flock to the lake seeking such species as kokanee salmon, rainbow trout, walleye and bass. Increased entrainment from Lake Roosevelt can only result in un-favorable publicity about the fishery and ultimately affect the economy of the region. Recent low lake levels shut down the Keller and Inchelium ferries causing great hardships for many people in the Keller and Inchelium communities, all within the boundaries of the Colville Indian Reservation. Increased time periods of exposed reservoir banks can increase airborne pollution and low lake levels adversely affecting the houseboat industry subsequently effecting tribal and state economies. Low lake levels also adversely affect the ability of fishermen and researchers to access the lake due to water levels below boat launch facilities. Creel census reports that indicate low fishermen success result in unfavorable fishing reports. This leads to reduced visits to the area resulting in impacts to the area in lower spending for tackle, gear, licenses, fuel, motel and dining activities.

Periods of unusual hydro-power operations such as occurred during 2001, greatly reduce water retention time. Lake Roosevelt has water retention times ranging from 8-65 days (Cichosz et al. 1999). Water retention times were calculated based on the monthly median lake elevation for the months of Jan., Feb., Mar., April, and May. Median elevations were estimated using the U. S. Corp of Engineers Columbia River Dart Table for Grand Coulee Lake. From that estimation average water retention times were calculated for 1996, 1997,1999 and 2001. High entrainment rates were documented during 96 and 97; both were banner water years. In 1999 the annual water cycle returned to a more normal mode, while 2001 is recording itself as one of the lowest on record.

Water Retention Time (Days) = Reservoir volume (kcfsd) / outflow (kcfs).

January      February    March    April     May

Table 1. Average Water Retention Times 1996-2001

| Ave. Ret. time (days) | January | February | March | April | May | Average for period |
| --- | --- | --- | --- | --- | --- | --- |
| 1996 | 24 | 26 | 24 | 31 | 16 | 24 |
| 1997 | 26 | 24 | 22 | 18 | 11 | 20 |
| 1999 | 31 | 32 | 22 | 24 | 39 | 30 |
| 2001 | 31 | 27 | 24 | 31 | 22 | 27 |

Table 2. Beginning Monthly lake elevations, Maximum monthly changes in feet, low elevations for respective year and refill start up data by year.

| Begin Month Elevation | January | February | March | April | May |
| --- | --- | --- | --- | --- | --- |
| 1996 | 1284 | 1278 | 1274 | 1244 | 1230 |
| 1997 | 1275 | 1263 | 1243 | 1232 | 1210 |
| 1999 | 1278 | 1264 | 1265 | 1246 | 1220 |
| 2001 | 1268 | 1241 | 1227 | 1222 | 1220 |
| Maximum change |  |  |  |  |  |
| 1996 | 6 | 30 | 29 | 14 | 3+ |
| 1997 | 12 | 20 | 10 | 22 | 41+ |
| 1999 | 14 | 5 | 19 | 26 | 20 |
| 2001 | 27 | 14 | 4 | 3 | 17 |
| Low ele. | 96/1227 | 97/1208 | 99/1213 | 01/1217 |  |
| Refill start | 96/5-10 | 97/5-9 | 99/5-21 | ¼-28 |  |

In general, high entrainment correlates to years where the retention times are shorter. Short retention times were recorded in 1996, 1997 and 2001 (Table 1). A longer retention time was recorded during 1999 while entrainment was considerably lower than in previous years (Table 12).

Zooplankton densities in Lake Roosevelt decrease during periods of short retention time (Underwood and Shields 1995) thus leading to a reduction in the primary food source for juvenile rainbow trout, kokanee, suckers and whitefish (Underwood 2000). The operation of Grand Coulee Dam during 2001 significantly reduced the production of primary food sources for juvenile fish and should be compensated for.

A forty-two month study of fish entrainment at Grand Coulee Dam determined that entrainment was substantial, ranging from 211,685 in 1999 to a high of 576,676 in 1996 (Table 12).

TABLE 12. Annual Entrainment Totals March 1999-September 1999.

| Power plant | 1996 | 1997 | 1998 | 1999 | Total |
| --- | --- | --- | --- | --- | --- |
| Left | 10,442 | 33,192 | 26,718 | 9,313 | 79,665 |
| Right | 27,316 | 32,811 | 50,706 | 19,741 | 130,574 |
| Third | 538,918 | 470,009 | 208,926 | 182,631 | 1,400,484 |
| Total | 576,676 | 536,012 | 286,350 | 211,685 | 1,610,721 |

Data collected by the Chief Joseph Kokanee Project from 1996 through September 1999 indicate that high rates of entrainment occur during lake refill periods following extreme draw-down periods (LeCaire. 1999). Entrainment rates during high water years has been greater than entrainment rates during low or normal water years (LeCaire 1999), (Table 10). Target strength data collected during a 42-month hydro acoustic study of entrainment at Grand Coulee Dam determined that many of the fish targets were of the size of recently released hatchery and net pen fish (Table 18) (BioSonics 1999). Further, data indicates that there is a close correlation between net pen and hatchery release strategies and entrainment peaks (Figure 9). Kokanee and rainbow trout were the dominant species encountered during a 42 month gill net study conducted in the forebay area of Grand Coulee Dam concurrent with the entrainment study (LeCaire, 1999).

TABLE 18. Table of acoustic target strength converted to approximate lengths in millimeters, Left Power Plant, 1999.

| Month | Acoustic size | Fish length |
| --- | --- | --- |
| January | -39.6 dB | 199 mm |
| February | -41.4 dB | 159 mm |
| March | -42.9 dB | 134 mm |
| April | -35.5 dB | 323 mm |
| May | 0 | 0 |
| June | -40.25dB | 183 mm |
| July | -39.9 dB | 190 mm |
| August | -42.7 dB | 136 mm |
| September | -42.8 dB | 135 mm |

Failure to replace the fish lost as a result of the emergency power operations will lead to a reduced sport and consumptive fishery and may lead to unfavorable economic conditions for the region.

The use of net pen technology has been clearly shown to increase the availability of fish to anglers  (Scholz et al. 1990) (Cichosz, 1998). Bonneville Power Administration currently embraces the use of this methodology by providing annual funding for two fish hatcheries that supply fish for the net pen operation and for direct seeding into the lake (Smith, per. Communication. 2001). The purchase and stocking of triploid steelhead trout  into Lake Roosevelt will overcome some of the loss and provide immediate relief to sport and subsistence fishermen. The increase of net pens, stocking and release will also help overcome some of the lost fishery opportunities both on an immediate and long term basis.

The use of triploid fish for stocking Lake Roosevelt will prevent any genetic contribution or dilution of the gene pool. Use of this stock will provide a larger biomass contribution to the fishery due to the accelerated growth from the net pen facility where they were raised. Finally, this stock is the only fish source readily available for purchase on short notice.

We estimate the total loss of fish at 61,212 fish at an approximate weight of 9,800 lbs (Smith. 2001, Personal communication).

To make calculations a little simpler we are assuming that we lost 10,000 pound of fish. Assuming a loss of 80 percent to entrainment or a retention of 20% that leaves 8 thousand pounds of fish to replace. Compounding the loss to the fishery is the early release (4 months early) and considerable lost growth. We estimate that the fish would have increased in weight 60% (Combs, 2001) to a lost weight of 32,000 pound. We are requesting compensation for 32 thousand lbs of fish. The weight allocation is based on the following, 24 thousand pound of legal size (12,000 2 lb fish) and 8 thousand lbs of fingerlings (136,000 @ 17/lb.)

Replacing the lost fish on a fish for fish basis only makes sense

Figure 1. Total monthly entrainment at all three power plants corrected for dam operations

Figure 6. Lake Elevation vs. Fish Entrainment 1996-1999.

Figure 9. Fish Release Numbers by Month vs. Entrainment, 1999.

Figure 10. Entrainment Numbers vs. Monthly Discharge (kcfs), 1999.

Work on this emergency proposal will begin as soon as we receive a favorable response from BPA. When an agreement is reached we will immediately contact the Columbia River Fish Farm so they can withhold the necessary complement size for immediate transfer to Lake Roosevelt. The legal size, triploid rainbow tout stock is currently available, fingerling sizes (17/lb) will be available when needed (Shallenberger, 2001).

Hatchery vehicles at the Colville Tribal Hatchery have nearly completed their spring planting schedule. An ok from BPA will help us secure the necessary fuel required to facilitate the additional planting. It will be necessary to plant the legal size fish ASAP due to the increasing water temperatures associated with summer time. We have the manpower and vehicles all we need is the fish and fuel.

Normal net pen seeding is normally done during the fall period. Following a favorable response we will make the necessary arrangements with the Columbia River Fish Farms to have 136,000 triploid rainbow trout available for the fall planting period. All work will be complete within one year except for some minor data analysis related to the floy tag/ performance phase.

c. Rationale and relationship to criteria for 2001 Action Plan For Fish in Response to Power System Emergency Solicitation of Proposals DM

This proposal addresses impacts to non-listed steelhead and resident fish species. There are no NEPA, ESA, or 404 concerns present in the Lake Roosevelt area that would be affected by this proposal.

Funding is sought for a single year one-time basis only. The project is consistent with the federal government’s trust and treaty obligations. All project work will comply with the Endangered Species Act and is consistent with the Northwest Power Act and applicable state laws.

d. Relationships to other projects

The Colville Tribe/Lake Roosevelt Emergency Fish Restoration Project will be tied to several Bonneville Power Administration funded projects that include the Lake Roosevelt Monitoring Project # 9404300. It is linked to the monitoring project in that we are proposing to study the success/failure of the triploid steelhead stock planted into Lake Roosevelt. This will be accomplished at no cost to Bonneville Power Administration other than the cost associated with the purchase of the tags and the labor to reimburse for the tagged fish lost as a result of operations. Primary data collection duties will be carried out by personnel of the Lake Roosevelt Evaluation Project.

?

The proposal is related to BPA funded hatchery projects, Spokane Tribal Hatchery #9104600, Sherman Creek Hatchery #9104700, and the Lake Roosevelt Net Pen Project #5521800. These projects provide kokanee salmon and rainbow trout for direct out planting and net pen projects funded on the lake. This project intends to replace fish lost from the net pen operations during emergency power generation operations at Grand Coulee with triploid steelhead purchased from the Columbia River Fish Farm.

The proposed project will be linked to the Rainbow Trout Habitat/Passage Improvement Project #19901800 by providing a mechanism to compare the performance of a triploid stock to wild and hatchery origin fish. Again, the majority of the work under this proposal will be accomplished by currently funded Bonneville projects at no additional cost to BPA.

e. Proposal objectives, tasks and methods

Goal: For Lake Roosevelt to continue to provide sport and consumptive fishing, and recreational opportunities for all. ?

Objective 1, Replace fish lost as a result of emergency hydropower operations with triploid steelhead for sport and consumptive use.

This objective is closely linked to criteria (4) “Fish stock relocation and out-planting”. During 2001 lake elevations were unusually drawn down for power generation needs due to the emergency power crisis.  Draw downs at Lake Roosevelt are part of the annual water cycle as the storage reservoirs are drawn down to make room for the upcoming spring snow melt water. In 2001 lake elevations were drawn down much earlier than normal and to a depth that would indicate a high water cycle; however, spring 2001 was a drought year where snow packs were less than 50% of normal and the annual water cycle is the second worse on record since 1944  (Table 2), (Columbia Basin Bulletin).

The early draw down forced net pen operators to release some net pens as much as two months early. On February 12, 2001 with lake elevations at ele. 1,233 msol, Gene Smith, Lake Roosevelt net pen coordinator was forced to release 52,212 rainbow trout from the net pens located at Hall Creek on the Colville Indian Reservation. The approximate weight of these fish was 7,811 pounds. The cause of the early release was due to the grounding of the nets that were caught on bottom debris. On May 6, 2001, Gene Smith was again forced to a early release of approximately 9,000 rainbow trout from net pens located at 7-Bays. The approximate weight of this release was 2000 pounds. 5000 of these fish had been tagged.

We are proposing to replace fish lost to emergency hydropower generation operations

on a fish for fish basis. We propose the total purchase of 32,000 lbs of fish from Columbia River Fish farms. The Columbia River Fish farm is privately owned and operated; located at Nespelem WA on the Colville Indian Reservation. We anticipate the purchase of 12,000 triploid rainbow trout (24,000 lbs) at approximate weight of 2 lbs each at $1.31/lb. These fish would be transported and planted in locations along the Colville Reservation side of the reservoir from Hall Creek south to Grand Coulee dam. An additional 8,000 lbs of fish at 17/lb (fingerlings) or 136,000 fish would be purchased for the net pen aspect of this proposal. This cost would be $81,600,00. These fish would be placed into the net pen operations and allowed to grow to compensate for lost growth opportunity resulting from early release of fish this spring.

Task 1. Purchase 32 ,000 pounds of triploid rainbow trout from Columbia River Fish Farm

Task 2. Transport 24,000 pounds of legal size (1.75-2 lb) fish to predetermined release sites along the Colville Indian Reservation boundary of lake Roosevelt and release

Task 3. Transport 8,000 pounds of fingerling triploid rainbow trout to net pen complexes

Objective 2, Purchase Net pen nets, floats, anchors, feed storage, etc.  install and operate for 8 months then turn nets over to the Lake Roosevelt Net Pen Project group to replace worn nets that need to be recycled.

Task 1. Purchase four, four net pen arrays from WDFW source.

Task 2. Transport and set up net arrays using currently owned equipment and vehicles

Task 3. Seed fingerling triploids to net pen complexes?

Objective 3. Monitor success/failure of planted steelhead in Lake Roosevelt using floy tags as marking indicators.

Since we are advocating out planting thousands of triploid steel head trout this may be an opportune time to assess their performance/success in Lake Roosevelt. Lake Roosevelt monitoring data indicates that floy tag recovery averages about 20 tags per 1,000 fish tagged in minimum lots of 10,000 fish. To accumulate a tag return of 400 fish then we would need to tag 20,000 steelhead. The cost for this effort would be fairly low.   A great deal of knowledge could be garnered regarding the performance of the fish. The use of triploid steelhead would eliminate any possible genetic introgression into the wild component or the current hatchery stock.

Tasks and Methods?

Objective 1.

Task A. Purchase triploid steelhead from Columbia River Fish Farms

Task B. Consult with Lake Roosevelt Fishery managers to develop schedule and

desired out planting locations.

Task C. Transport fish to desired location by schedule and release into lake.

Task D. Transport and release fish into net pens for later release.

Objective 2.

Task A. Purchase Net Pen material as per requirements of the LR Net Pen Program

Task B. Plant triploid steelhead into net pens at needed locations

Task C. Operate Net pens for eight months

Task D. Release fish at pre-determined date and turn new net complexes over to Net

Pen program.

Objective 3. Monitor success/failure of triploid stock as compared to wild/hatchery stock.

Task A. Purchase 136,000 triploid steelhead for  net pen project.

Task B. Tag (using floy tags) 20,000 of the test  group and release into Lake Roosevelt                                                                  in conjunction with other groups for comparison.

Task C. Collect and analyze data to determine success/failure.

f. Facilities and equipmentDM

It would not be necessary to purchase additional equipment or facilities to conduct this project. The Sherman Creek and Spokane Tribal Hatcheries are presently operated by Bonneville Power Administration. Current personnel, equipment, computers

etc would be used to complete the objectives. There are no new position expenses associated with this proposal. Expenses are related to the following; purchase of triploid fish, purchase of net pen complexes, fuel costs associated with transportation, contract administration, floy tags and reimbursement for labor associated with the duplication of tagging effort.

It would be necessary to purchase four new net pen complexes consisting of four pens each for a total of sixteen pens. At the end of this phase of the project these pen complexes would be given over to the current net pen project operators. A portion of the current net pen complex is in need of replacement with newer, stronger, safer nets and walkways. The purchase under this emergency solicitation process would remove some of the cost to BPA under normal maintenance activities.

g. ReferencesDM

BioSonics Inc. 1999. Annual and Final Report on fish entrainment at Grand Coulee Dam 1996-1999. N Leary Way, Seattle WA.

Cichosz, T. A., J. P. Shields, and Keith Underwood. 1999. Lake Roosevelt       Monitoring/data collection program: 1997 annual report. Report to U. S. Department of Energy, Bonneville Power Administration, Division of Fish and Wildlife. Portland OR. Contract # 88-63.

Combs, Mitch. 2001. Personal Communication Washington Department of Fish and Wildlife, Sherman Creek Hatchery Manager.  Kettle Falls WA..

LeCaire, R. 1999. Draft Chief Joseph Kokanee Enhancement project: 1999 annual report and final report on entrainment. Report to U.S. Department of Energy, Bonneville Power Administration, Division of Fish and Wildlife, Project number: 9501100.

LeCaire, Richard and Joe Peone. 1991. Lake Roosevelt Rainbow Trout Habitat/Passage Improvement Project # 19901800 Final Report Phase I  Report to U. S. Dept. of Energy, Bonneville Power Administration, Division of Fish and Wildlife, Contract No. DE-79-90BP08120. Portland OR.

LeCaire, Richard. 2000. Rufus Woods Subbasin Summary. Intermountain Province

Report Prepared for the Northwest Power Planning Council under guidance

from the Columbia Basin Fish and Wildlife Authority, Portland OR.      .

Marco, Jerry. 2001. Personal communication. Confederated tribes of the Colville Indain Reservation. Fish and Wildlife Department, Nespelem WA. 99155

Shallenberger, Ed.  2001. Personal communication. Columbia River Fish Farms.  Nespelem WA.

Smith, Gene. 2001. Personal communication. Lake Roosevelt Net Pen Coordinator. Davenport WA.

Sullivan, Colleen. 1999. Final Report on Entrainment at Grand Coulee Dam. Report prepared for Colville Confederated Tribes, Fish and Wildlife Department.

Underwood, Keith. 2000. Lake Roosevelt Subbasin Summary. Intermountain Province.

Report Prepared for the Northwest Power Planning Council under guidance

from the Columbia Basin Fish and Wildlife Authority, Portland OR.

Section 4. Key personnel?

All personnel that will be associated with this project are currently employed by the various tribes and agencies. These people meet the minimum educational and experience

requirements of Bonneville Power Administration as per contractual agreement. All of the tribes and agencies have minimum job requirements that were met upon initial hiring.