Project ID:	25034

Title:	Develop a Nutrient/Food-Chain Management Tool for Watershed-River Systems

# Section 9 of 10. Project description

a. Abstract

Growth and abundance of salmon are directly affected by the dynamic interaction of nutrients and the associated food-chain. An integrated analysis tool that can simulate the transport and uptake of the added nutrients, and the resulting effect on juvenile fish growth, spawning escapement and population response is currently unavailable. Development of this physically-based modeling system will provide an important tool to help managers decide whether to initiate nutrient supplementation in a particular watershed system.  Such a tool will also be useful for determining the appropriate timing and quantity of nutrient supplementation for the system.  The proposed work will link state-of-the art watershed and river models together with nutrient and food-chain components to form a integrated analysis system. The outputs from the modeling system would be presented in the form GIS data layers that can be directly used by management agencies to prioritize areas that would benefit from nutrient enhancement.

b. Technical and/or scientific background

Numerous studies suggest that the spawning of anadromous, semelparous fish (fish that die after they spawn) is an important nutrient transport mechanism, especially for the salmon of the Pacific Northwest (e.g. Winter et al. 2000; Bilby et al. 1996; Kline et al. 1990).  These fish carry marine-derived nutrients (MDN), primarily carbon, nitrogen and phosphorus, to the nutrient-poor freshwater spawning sites in the upper reaches of rivers and streams.  After spawning, the carcasses, excrements, and gametes are consumed and broken down by fish, upland scavengers (Cederholm et al. 1989), invertebrates, bacteria, and fungi.  The released nutrients are also sorbed onto bottom sediments (Bilby et al. 1996) and taken up by primary producers such as phytoplankton and periphyton (Richey et al. 1975).  This pulse of MDN can enhance productivity and improve the growth and survival of age-0 and juvenile salmon (Bilby et al. 1998;  Bilby et al. 1996; Larkin and Slaney 1997).

Presently, however, this source of MDN is greatly diminished because of declines in salmon spawning over the last century.  Some researchers estimate that MDN deposition by spawning salmon has decreased to 6 - 7 percent of historical levels (Gresh et al. 2000).  The resulting nutrient deficit adversely impacts the ecology of upland and aquatic biota, including the growth and survival of parr and smolts.  Such an effect creates a negative feedback where diminished spawning reduces MDN, which compromises smolt survival, further diminishing return adult spawning numbers.

In response to these findings, Washington, Oregon and Canada have all implemented nutrient enrichment programs.  Washington and Oregon both have rapidly growing enrichment programs that involve placing hatchery salmon carcasses in spawning areas. Canada is experimenting with artificial substitutes.  Bio-Oregon, an Oregon company that manufactures fish pellets, is working with the Washington Department of Fish and Wildlife (WDFW) on the development of salmon carcass analogs which may provide salmon derived MDN in the form of an easily transported, disease-free, dried concentrate.

There are many considerations associated with nutrient enrichment programs such as disease, water quality, cost/benefit tradeoffs, and the timing, amount, and location of nutrient introductions.  The nutrient budget in a given channel is driven by a number of complex interrelated physical and biological processes including watershed hydrology, geology, channel hydraulics, vegetation, temperature regime, solar radiation influx, and channel food web.  Ideally, a program would involve a systematic, science-based approach for identifying the rivers and streams likely to benefit most from added nutrients, then adding the appropriate nutrients in the appropriate amounts at the appropriate time.  A model that simulates the breakdown, transport, immobilization and uptake of the added nutrients, and the resulting effect on smolt growth, spawning escapement and population response would be useful for identifying the best sites and estimating these “appropriate” amounts.  We propose a watershed model that incorporates watershed hydrology, channel hydraulics, fate and transport of dissolved constituents, and in-channel food web dynamics.  Such a model could function as a design tool for these watershed management programs.  It could also help identify and therefore prevent unintended adverse effects, such as water quality deterioration.

This project will build upon a foundation provided by our existing watershed and river modeling systems. These models, DHSVM (Distributed Hydrology Soil Vegetation Model) and MASS1 (Modular Aquatic Simulation System 1D), will be linked together to form a coupled modeling system. These models have been used for several watersheds and rivers in the Pacific Northwest. The basic capabilities of each model and representative applications are described below.

DHSVM (Wigmosta et al., 1994; Storck et al. 1998; Wigmosta and Lettenmaier 1999; Wigmosta and Perkins, 2001) is a physically based distributed parameter model that provides an integrated representation of watershed processes at the spatial scale described by digital elevation model (DEM) data.  DHSVM has been utilized in a number of research activities involving hydrologic analysis and modeling (Wigmosta et al., 1994; Haddeland and Lettenmaier, 1995; Kenward and Lettenmaier, 1997; Wigmosta and Lettenmaier, 1999; Westrick et al., 2000).  The model has also been used to study the interactions between climate and hydrology (Wigmosta et al., 1995; Arola and Lettenmaier, 1996; Nijssen et al., 1997), and the potential impacts of climate change on water resources in the American River  and the Yakima subbasin as a whole (Leung et al., 1996; Leung and Wigmosta, 1999; Wigmosta and Leung, 2001).  There has been significant use of the model for basic and applied research concerning forest management activities on watershed processes (Storck et al., 1995, Lamarche and Lettenmaier, 1998; Storck et al., 1998; Storck et al., 1999; Bowling et al., 2000; Storck, 2000; Bowling and Lettenmaier, 2001; Wigmosta and Perkins, 2001).

DHSVM provides an integrated representation of watershed processes at the spatial scale described by digital elevation model (DEM) data. It includes a multi-layer canopy model for evapotranspiration, energy balance models for canopy snow interception and ground snowpack processes, a multi-layer rooting zone model, and subsurface, surface, and channel flow modules.  The modeled landscape is divided into computational grid cells centered on DEM elevation nodes.  Digital elevation data are used to model topographic controls on absorbed short-wave radiation, precipitation, air temperature, and down-slope water movement.  At each time step, the model provides a simultaneous solution to the energy and water balance equations for every grid cell in the watershed.  Each cell exchanges water with its adjacent neighbors, resulting in a three-dimensional redistribution of surface and subsurface water across the landscape.

MASS1 is a one-dimensional, unsteady hydrodynamic and water quality model for river and reservoir systems. It is a physically-based model that numerically solves the partial-differential equations representing the conservation of water mass, momentum, thermal-energy, and constituent mass in a channel. MASS1 has been used for a variety of river modeling studies in the Pacific Northwest. The hydrodynamic component of MASS1 was used to simulate unsteady water elevations in the Hanford Reach in a fish stranding study (Evaluation of Juvenile Fall Chinook Stranding on the Hanford Reach, BPA Project Number 199701400). The effect of Lower Snake River Dams on water temperatures was studied by simulating 35-year period for impounded (post-dam) and unimpounded (pre-dam) river conditions (Perkins and Richmond, 1999). To evaluate different gas abatement alternatives, MASS1 was used to simulate temperature changes, and the transport and fate of dissolved gas produced by spillway discharges from Lower Snake and Columbia River dams (Richmond et al., 2000).

MASS1 simulates averaged processes in branched channel systems at discrete cross-sections (i.e., one-dimensional). Cross-sections can be located at variable distances along the channel and be defined using survey data or estimated from DEM data. The model accepts time varying inflows at points (tributaries) or as lateral inflow/outflow along a channel reach (groundwater influx). Channel resistance is modeled using a Manning or Chezy roughness coefficient. Hydraulic structures such as dams, weirs, and culverts can be included in a channel network. Heat exchange processes at the water surface are modeled using standard parameterizations; meteorological conditions can be assigned on a zonal basis using multiple weather stations or from DHSVM output. The model includes a general mass transport equation solver that allows any number of constituents (up to computer memory limits) to be simulated by specifying individual source/sink processes.

c. Rationale and significance to Regional Programs

The rational for nutrient enhancement is that spawning escapement numbers are a fraction of historical values, and as a result, the supply of MDN to infertile streams through the decay of salmon carcasses is reduced.  This rational is becoming fairly well known and accepted.  Evidence of this acceptance is shown in the Yakima Subbasin Summary where nutrient concentration and mutualism are listed as Current Limiting Factors.  Nutrient enrichment programs can potentially provide a relatively low-cost means to restore the mutualistic ecological interaction between salmon carcasses and aquatic communities, and thus increase smolt growth, survival and natural spawning to a level that eliminates the need for nutrient enrichment over the long-term.  Because of the potential benefits that nutrient enhancement offers, such programs are becoming more common.  The Yakima Subbasin Summary alone referred to nutrient enhancement projects in the American, Bumping, Little Naches, and Manastash, as well as six tributaries in each of the Yakima, Klickitat and Salmon River Basins.

In light of the growing interest in nutrient enhancement programs, a model that can simulate the subsequent local and downstream effects has obvious significance. Such a model could eliminate the costs and risks associated with a trial and error implementation of watershed-scale intervention programs.  A direct quote from Section 7 of the North West Power Planning Counsel-Fish and Wildlife Plan articulates the need for a scientific approach. “Our history is replete with well-intentioned, but ill-informed actions compounding the problems they were intended to solve.”  We propose this watershed-scale model as a means to ensure that well intentioned, watershed scale intervention measures, such as nutrient enrichment, are implemented using a systematic and scientifically defensible approach.  The model would allow simulation of multiple management options at the subbasin scale.  For example, the model could simulate the effects of nutrient enrichment, grazing, logging, fire, and flow alteration, either singly or in various combinations.  The simulation results will then help federal, state or local watershed managers identify the options likely to provide the most cost effective benefits to smolt survival, spawning escapement, ecosystem productivity and function, water quality and the salmonid population as a whole.

d. Relationships to other projects

In the Columbia Plateau Region, numerous nutrient enhancement programs and studies are proposed or in progress. This watershed-scale nutrient model will ultimately provide a scientific means of assessing the potential benefit of proposed projects and whether the potential benefits of existing projects are being realized. The previously mentioned WDFW nutrient enhancement programs in the Yakima Subbasin on the American, Bumping and Little Naches Rivers relate particularly well to this work because the DHSVM model has already been configured there and the necessary watershed data gathered.  Additionally, the field study proposed herein would provide an extensive evaluation of the ecological response in the Yakima Subbasin to these ongoing nutrient enhancement programs.  The knowledge gained from this proposed field component will greatly contribute to the body of information presently available on nutrient enhancement in the inland watersheds of the Columbia Basin.

Once the model is calibrated with field data, it will provide a valuable analysis tool for assessing the interactions between the watershed-river nutrient cycle and salmon production. It could be used to perform a baseline assessment of the nutrient cycle in a watershed to which various nutrient supplementation schemes could be compared and evaluated.  This sort of capability will complement any of the numerous existing or proposed nutrient assessment or enhancement projects. The proposed analysis system links physically based watershed and river system models. Subsequently, it can be used to provide information (runoff volumes, channel velocities, water depths, nutrient loads, sediment loads, etc.) to habitat restoration/enhancement projects (for example, BPA Project Numbers 199705600 and 21017) as well as watershed-scale projects.  With proper coordination, the GIS format of the output information will support and link with other GIS productivity models and databases.  The stream temperature and nutrient-cycle components of the model will also be useful to other existing or proposed projects that focus on the many water quality issues in the Yakima Subbasin.

e. Project history (for ongoing projects)

NA

f. Proposal objectives, tasks and methods

The overall objective of this work is to develop a state-of-the-art, integrated modeling system that can describe the complex interactions between the physical and nutrient-cycle processes in watersheds and rivers and their ultimate impact on salmon production.

- Model Development

The existing watershed and river system models, DHSVM and MASS1, will be enhanced to include nutrient cycling and sediment transport.

  - Watershed

DHSVM will be enhanced to simulate the two hydraulic processes generally responsible for surface soil erosion: soil detachment by rainfall, and hydraulic entrainment of sediment.  The rate of soil detachment by rainfall is a function of surface cover, soil properties, and rainfall intensity.  The amount of soil removed through hydraulic entrainment is dictated by soil characteristics and local surface water hydraulics.  There are several physically-based approaches for modeling these erosional processes.  The U.S. Forest Service, the Agricultural Research Service, the National Resource Conservation Service, and the Bureau of Land Management are developing new methods for predicting soil erosion under the Water Erosion Prediction Project - WEPP (Flanagan and Nearing, 1995).  The WEPP approach for rainfall detachment and hydraulic entrainment will be incorporated in the modeling system.  Information on vegetation and soil properties will be supplied for each hillslope grid cell and road reach using ARC/INFO.

We will adapt a simple nutrient budget module that will initially consider only nitrogen and phosphorous (Bouraoui and Dillaha, 2000).  These are the two most important nutrients in forest ecosystems.  The processes of nitrification, plant uptake, denitrification, bacterial degradation of organic matter, and export will be represented in the model.  Plant uptake of nutrients will consider uptake both through the roots and leaves.  Nitrification involves the transformation of ammonium into nitrate and will require accounting for nitrogen in distinct albeit linked forms.  Denitrification is the process whereby certain organisms under anaerobic conditions can transform nitrate into nitrogen gas.  Denitrification will only be considered in watershed grid cells representing wetland areas.  Bacterial degradation occurring in the litter layer transforms nitrate bound up in organic matter back into ammonium.  Soluble forms of nitrogen and phosphorous will be allowed to migrate among the vertical compartments (canopy, litter layer, and each of the soil layers) and horizontally between grid cells in saturated subsurface flow.  Both nutrients attached to sediments and soluble nutrients will be allowed to migrate with surface flows.  Nutrient budgets will be estimated for the various soil horizons, the litter layer, stream, lake, wetlands, and the forest canopy.  This component of the modeling system will allow assessment of the cumulative impacts of land management practices on nutrient contaminant transport.

  - River

MASS1 will receive water, sediment, and nutrient influxes from DHSVM. DHSVM will also provide incoming water temperature and meteorological data to calculate heat fluxes at the water surface based on air temperature, wind speed, canopy and cloud cover, and relative humidity. 

In streams with steep gradients, the channel routing will be done by solving the full St. Venant equations (continuity and momentum) equations using the method proposed by Meselhe, et. al. (1997) which allows for the simulation of  mixed subcritical and supercritical flow regimes. In this method we allow for the channel friction factor to be a fixed value or a function of sediment discharge.

The sediment transport model will be formulated to allow simulation of nonuniform sediment mixtures and channel aggradation/degradation (Rahuel et al., 1989). Both bedload and suspended load transport will be included in the model using the methods proposed by Holly and Rahuel (1990). Changes in channel bed elevation are computed using the sediment-continuity equation together with a sediment-discharge predictor equation. Many sediment-discharge predictor equations are available in the literature and have been used with varying degrees of success. The modular model development approach will allow the results of ongoing research on sediment-transport in high-gradient streams to be incorporated into the modeling system. For example, the model will be formulated to allow the user to select from several common choices such as the Meyer-Peter Mueller equation. The mobile-bed simulations can be done using a fixed friction factor or a sediment-discharge dependent friction factor, such as proposed by Karim and Kennedy (1981). Empirical relations to simulate the effect of channel armoring will also be included the model.

The nutrient cycle component will be formulated using the relationships in the EPA QUAL2E model (Brown and Barnwell, 1987). This includes nitrogen, phosphorus, dissolved oxygen, and algal interactions. The mathematical relationships for these processes will be included in the MASS1 model.

  - Food Chain

A dynamic food-chain model component will be added to the MASS1 model. The food-chain model will be formulated along the lines presented by Yang and Sykes (1998) as a coupled system of rate equations describing the linkages between the trophic levels. This formulation will be extended to include direct consumption of salmon carcasses and eggs by juvenile salmon based on observational data by Bilby, et al. (1998). The mass-balance component for salmon carcasses would include direct transport by the stream, decomposition, consumption by juveniles and other aquatic species, and consumption by non-aquatic species (mammals and birds).

  - Data Transfer

To be successful, the project must present the modeling system output in ways that can be readily used by management agencies. A geo-spatial data manager will be developed to provide a mechanism for data transfer to external users.  A customized Arc View project will be developed as a tool to archive, display, query, and output information from the models.  Data sets, field-measured and simulated, will be assembled, reformatted, and integrated in an easy-to-use interface.  Examples of the data include instream habitat information, stream geomorphology metrics, substrate composition, stream cross-sections, historical photography, digital airborne videography, endangered fish collection and tracking information, water quality parameters, and nutrient loads.  The customized software will also feature the ability to choose the desired geographic area of interest such as a predefined geomorphic stream reach, desired river mile range, or predefined landmarks, and the ability to quickly generate and print map layouts. The software will be designed in such a way that data integrated into the project could be selected for output to formats used by other applications for further processing and analysis.

  - Preliminary Calibration and Testing

Preliminary testing will be conducted by applying the linked modeling system to simple test cases designed to isolate individual model components.  The performance of these components can then be verified by comparison with the expected results.  For testing purposes, the model will initially be calibrated to the field experiments documented by Bilby, et al (1996), which were conducted on tributary streams of the Snoqualmie River.  Measurements of stable isotopes of nitrogen and carbon were used to trace MDN input from spawning salmon through the food chain of the stream. Sampling was done on four streams; two streams had spawning salmon and two did not.

- Data Gathering

In this task we will gather the data necessary to apply the models to the selected test cases. The DHSVM watershed hydrology model is already configured for the American and Yakima River Basins, with the latter being at a coarser scale. Much of the necessary hydrologic and meteorological data is available through various sources such as stream gage stations, weather stations, and Earth Info discs. The data necessary to calibrate and test the nutrient model include carcass decay rates, river discharges, river stages, invertebrate utilization of added nutrients, juvenile fish and fry utilization of added nutrients, fish growth and survival rates, and downstream nutrient levels. Some data of this type is available for coastal streams in western Washington (Bilby et al., 1996; Bilby et al., 1998), however these systems differ from those in eastern Washington in a number of ways including geology, temperature regime, snowpack characteristics, baseline productivity, invertebrate community composition, and water quality to name a few. Because of these differences, it is important to calibrate and test the model using data appropriate for the Yakima Subbasin. Subsequently, data specific to the current nutrient enhancement programs on the American, Bumping and Little Naches Rivers will be provided by WDFW under a subcontract.

  - Watershed Hydrology and Flow Model

No field work is planned for the watershed model task because DHSVM has already been applied to the Yakima and American River basins. The required physical data include elevations, soil types, vegetation, sediment types, and meteorology. The resulting watershed yield values from DHSVM and the hydraulic results from MASS1 will be verified against available stream gage information.

  - Nutrient Model

The Washington Department of Fish and Wildlife will be subcontracted to perform a thorough evaluation of the nutrient enhancement projects in place on the American, Bumping, and Little Naches Rivers. The objective of the work will be to identify the carcass decay rates, water chemistry, uptake of salmon carcass nutrients by stream organisms, and fish growth.   This evaluation will involve a comprehensive combination of field data collection, sampling, and lab analysis for numerous treatment and control sites.

The carcass breakdown rate will be monitored and recorded.  Water chemistry analysis will be conducted to track levels of phosphorus and nitrogen containing compounds upstream of carcass introduction sites, on site and downstream.  Invertebrate drift and benthic samples will be collected and used to estimate productivity, taxonomic composition, and food availability for fish.  The population densities, weight, and length of juvenile fish will be estimated from fish samples.  The stomach contents of some fish will be examined to determine the extent of direct feeding on carcasses.  Stable isotope analysis will be conducted on portions of the invertebrate and fish samples to determine the percentage of MDN.  Percent MDN, specifically carbon and nitrogen, is commonly used to estimate the proportion of nutrients derived from the salmon carcasses versus those from other sources.

- Model Application

Ultimately the model will be calibrated and tested by applying it to the data provided by WDFW on the current nutrient enhancement programs in the Yakima Subbasin. The integrated model will be calibrated to using data from one of the American, Bumping and Little Naches Rivers.  The model will then be tested by applying it to one or more of the treatment and control rivers for the time periods that samples were collected during 2002.  Output variables that can be compared to empirical data include watershed yield, in-channel velocities, nutrient concentrations, uptake of MDN by aquatic invertebrates and fish, and the expected increases in individual size and/or abundance of the invertebrate and fish populations.

- Reporting

The methods and results from the project will be documented and made available in several forms. A final report will be prepared and made available in a PDF file format that can be downloaded from the web. In addition to the report, individual GIS data layers (ArcView shape files) and text files of model input conditions and simulation results will also be made available for download over the web. A digital summary of the project will be provided in the form of an ArcView project. The project results will also be documented in the form of one or more papers submitted for publication in a peer-reviewed archival journal. We also plan to present our work at a regional and national conference.

g. Facilities and equipment

The work on this project will be conducted at the Pacific Northwest National Laboratory in Richland, Washington. A broad range of computing hardware and software are available for use in conducting the project work. Hardware includes single and multiple-processor computer workstations (Windows and Linux), tape and DVD-RAM backup systems, and color printers. Software includes compilers (FORTRAN90 and C/C++), GIS software (Arc/Info, ArcView), and model/data visualization software (TecPlot, FieldView).  The existing computing environment is adequate to meets the needs of the proposed project.

h. References

| Reference (include web address if available online) | Submitted w/form (y/n) |
| --- | --- |
| Arola, A., and D. P. Lettenmaier, Effects of subgrid spatial heterogeneity on GCM-scale land surface energy and moisture fluxes, J. Climate, 9, 1339-1349, 1996. | n |
| Bilby, R.E., B.R. Fransen, and P.A. Bisson. 1996. Incorporation of nitrogen and carbon from spawning coho salmon into the trophic system of small streams: evidence from stable isotopes. Can. J. Fish. Aquat. Sci., 53: 164-173. | n |
| Bilby, R.E., R.R. French, P.A. Bisson, and J.K. Walter. 1998. Response of juvenile coho salmon (Oncorhynchus kisutch) and steelhead (O. mykiss) to the addition of salmon carcasses to two streams in southwestern Washington, USA. Can. J. Fish. Aquat. Sci., 55: 1908-1918. | n |
| Bouraoui, F. and T.A. Dillaha. 2000. ANSWERS-2000: Non-point-source nutrient planning model, J. Env. Engr., 126(10), 1045-1055. | n |
| Bowling, L., P. Storck, and D. P. Lettenmaier, Hydrologic effects of logging in Western Washington, Water Resources Research, in review, 2000. | n |
| Bowling, L., and D. P. Lettenmaier, The effects of forest roads and harvest on catchment hydrology in a mountainous maritime environment, in Influence of Urban and Forest Land Use on the Hydrologic-Geomorphic Responses of Watersheds, M.S. Wigmosta and S.J. Burges, eds., AGU Water Science and Applications Series, 2, in press, 2001. | n |
| Cederholm, C.J., D.B. Houston, D.L. Cole, and W.J. Scarlett. 1989. Fate of coho salmon (Oncorhynchus kisutch) carcasses in spawning streams. Can. J. Fish. Aquat. Sci., 46: 1347-1355. | n |
| Gresh, T., J. Lichatowich, and P. Schoonmaker. 2000. An estimation of historic and current levels of salmon production in the Northeast Pacific Ecosystem. Fisheries, 25: 15-21. | n |
| Haddeland, I., and D.P. Lettenmaier, Hydrologic modeling of boreal forest ecosystems, Water Resource Series, Technical Report 145, Dept. of Civil Engineering, University of Washington, 1995. | n |
| Heathwaite, L.H., 1995, Sources of eutrophication: hydrologic pathways of catchment nutrient export, in Man’s Influence on Freshwater Ecosystems and Water Use, G. Petts ed., IAHS Publ. 230, 161-175. | n |
| Holly, F.M. and J.L. Rahuel, 1990, New numerical/physical framework for mobile-bed modeling, J. Hyd. Res., vol. 28, No.4, p. 401. | n |
| Karim, M.F. and J.F. Kennedy, 1981, Computer-based predictors for sediment discharge and friction factor of alluvial streams, IIHR Report No. 242, Univ. of Iowa, Iowa City, IA. | n |
| Kenward, T., and D. P. Lettenmaier, Assessment of required accuracy of digital elevation data for hydrologic modeling, Water Resource Series, Technical Report 153, Dept. of Civil Engineering, University of Washington, 1997. | n |
| Kline, T.C. Jr., J.J. Goering, O.A. Mathisen, P.H. Poe, and P.L. Parker. Recycling of Elements transported upstream by runs of Pacific Salmon: I. 15 N and 13C evidence in Sashin Creek , Southeastern Alaska. Can. J. Fish. Aquat. Sci., 47: 136-144. | n |
| Lamarche, J., and D. P. Lettenmaier, Forest road effects on flood flows in the Deschutes river basin, Washington, Water Resource Series, Technical Report, Dept. of Civil Engineering, University of Washington, 1998. | n |
| Larkin, G. A., and Slaney, P.A. 1997. Implications of trends in marine- derived nutrients flow to south coastal British Columbia salmonid production. Fisheries, 22: 16-24. | n |
| Leung, L.R., M.S. Wigmosta, S.J. Ghan, D.J. Epstein, and L.W. Vail, Application of a subgrid orographic precipitation/surface hydrology scheme to a mountain watershed, J. Geophysical Research, 101 (D8), 12,803-12,817, 1996. | n |
| Leung, L.R., and M.S. Wigmosta, Potential climate change impacts on mountain watersheds in the Pacific Northwest, J. Amer. Water Resour. Assoc., 35 (6), 1463-1471, 1999. | n |
| Meselhe, E.A., F. Sotiropoulos, and F. M. Holly Jr. Numerical simulation of transcritical flow in open channels. J. Hydraul. Eng., 123(9):774-783, September 1997. | n |
| Nijssen, B., I. Haddeland, and D. P. Lettenmaier, Point evaluation of a surface hydrology model for BOREAS, J. Geophysical Research, 102, 29,367-29,378, 1997. | n |
| Perkins, W.A. and M.C. Richmond. 1999. Long-term, One-Dimensional Simulation of Lower Snake River Temperatures for Current and Unimpounded Conditions. Report submitted to U.S. Army Corps of Engineers, Walla Walla District. Pacific Northwest National Laboratory, Richland, Washington. | n |
| Rahuel, J.L., F.M. Holly, J.P. Chollet, P.J. Belleudy, and G. Yang, 1989, Modeling of riverbed evolution for bedload sediment mixtures, J. Hyd. Engr, vol. 115, No. 11, p. 1521. | n |
| Richmond, M.C., W.A. Perkins, and Y. Chien. 2000. Numerical Model Analysis of System-wide Dissolved Gas Abatement Alternatives. Report submitted to U.S. Army Corps of Engineers, Walla Walla District. Battelle Pacific Northwest Division, Richland, Washington. | n |
| Ritchy, J.E., M.A. Perkins, and C.R. Goldman. 1975. Effects of kokanee salmon (Oncorhynchus nerka) decomposition on the ecology of a subalpine stream. J. Fish. Res. Board. Can. 32: 817-820. | n |
| Storck, P., D. P. Lettenmaier, B. A. Connelly, and T. W. Cundy, Implications of forest practices on downstream flooding: Phase II Final Report, Washington Forest Protection Association, TFW-SH20-96-001, 1995. | n |
| Storck, P., L. Bowling, P. Wetherbee, and D. P. Lettenmaier, An application of a GIS-based distributed hydrology model for the prediction of forest harvest effects on peak streamflow in the Pacific Northwest, Hydrologic Processes, (12), 889-904, 1998. | n |
| Storck, P., T. Kern, and S. Bolton, Measurement of differences in snow accumulation, melt and micrometeorology due to forest harvesting, Northwest Science, (73), 87-100, 1999. | n |
| Storck, P., and D. P. Lettenmaier, Predicting the effect of a forest canopy on ground snow accumulation and ablation in maritime climates, in Troendle, C, Ed., Proc. 67th Western Snow Conf., Colorado State University, Fort Collins, 1-12, 1999. | n |
| Storck, P., Trees, snow and flooding: an investigation of forest canopy effects on snow accumulation and melt at the plot and watershed scales in the Pacific Northwest, Water Resource Series, Technical Report 161, Dept. of Civil Engineering, University of Washington, 2000. | n |
| Westrick, K. J., C. F. Mass, B. Nijssen, D. McDonnal, P. Storck, and D. P. Lettenmaier, Description of a high resolution hydrometeorological forecast system configured for real-time application, Weather Analysis and Forecasting, in review, 2000. | n |
| Wigmosta, M.S., L. W. Vail, and D. P. Lettenmaier, A distributed hydrology-vegetation model for complex terrain, Water Resources Research, 30 (6), 1665-1679, 1994. | n |
| Wigmosta, M.S., L.R. Leung, and E. Rykiel, Regional modeling of climate-terrestrial ecosystems interactions, J. Biogeography, (22), 453-465, 1995. | n |
| Wigmosta, M.S., and D.P. Lettenmaier, A comparison of simplified methods for routing topographically-driven subsurface flow, Water Resources Research, 35 (1), 255-264, 1999. | n |
| Wigmosta, M.S. and W.A. Perkins, Simulating the effects of forest roads on watershed hydrology, in Influence of Urban and Forest Land Use on the Hydrologic-Geomorphic Responses of Watersheds, M.S. Wigmosta and S.J. Burges, eds., AGU Water Science and Applications Series, 2, in press, 2001. | n |
| Wigmosta, M.S. and L.R. Leung, Potential impacts of climate change on streamflow and flooding in forested basins, in The influence of Environmental Change on Geomorphological Hazards in Forested Areas, R.C Sidle and M. Chigira, eds., Centre for Agriculture and Biosciences International, in review, 2001. | n |
| Yakima Subbasin Summary (Draft), Prepared for the Northwest Power Planning Council. Available at: http://www.cbfwa.org/files/province/plateau/subsum.htm | n |
| Yang, M.D. and R.M. Sykes, Trophic-dynamic modeling in a shallow eutrophic river ecosystem, Ecological Modeling, 105, 129-139, 1998. | n |

# Section 10 of 10. Key personnel

The key personnel for this project are:

- Marshall C. Richmond, Staff Engineer, Principle Investigator for River Simulation Component (0.13 FTE)

- Mark S. Wigmosta, Senior Research Engineer, Principle Investigator for Watershed Simulation Component (0.17 FTE)

- Gregory R, Guensch, Engineer, Task Leader for River Simulation (0.30 FTE)

- Scott Waichler, Senior Research Scientist, Task Leader for Watershed Simulation (0.31 FTE)

- Geoffrey McMichael, Research Scientist, Task Leader for Nutrient and Fish Component (0.12 FTE)

- Todd N. Pearsons, WDFW Research Scientist, Principal Investigator for Field Component (0.12 FTE)

- Other (0.31 FTE)

One-page resumes for these individuals follow.

Marshall C. Richmond

Education

Ph.D.,  Civil and Environmental Engineering, University of Iowa, 1987

M.S.,  Civil and Environmental Engineering, Washington State University, 1983

B.S.,  Civil and Environmental Engineering, Washington State University, 1982

Experience

1994-present	Staff Engineer, Pacific Northwest National Laboratory
1991-1994	Assistant Professor, Civil and Environmental Engineering, Washington State University
1987-1991	Research Engineer, Pacific Northwest National Laboratory
1983-1987	Exxon Fellow and Research Assistant, Iowa Institute of Hydraulic Research, University of Iowa

Expertise

Dr. Richmond’s areas of expertise are in the development and application of computational models of hydrodynamics, sediment transport, and contaminant transport in environmental systems, computational fluid dynamics (CFD), physical modeling of hydraulic structures, and fisheries engineering. Dr. Richmond is the developer of the MASS1 (Modular Aquatic Simulation System) one-dimensional and MASS2 two-dimensional hydrodynamics and water quality computer models. He is also leading the development of an individual model for fish called FINS (Fish Individual-based Numerical Simulator).

Selected Publications

Scheibe, T.D. and M.C. Richmond. 2000. Fish Individual-based Numerical Simulator (FINS): A particle-based model of juvenile salmonid outmigration and dissolved gas exposure history in the Columbia River Basin. Accepted for publication in Ecological Modeling.

Rakowski, C.L. and M.C. Richmond. 2000. Bonneville Project Adult Migrant Fallback Study. Proceedings of the HydroVision 2000 Conference. Charlotte, NC.

Richmond, M.C., W.A. Perkins, and Y. Chien. 2000. Numerical Model Analysis of System-wide Dissolved Gas Abatement Alternatives. Report submitted to U.S. Army Corps of Engineers, Walla Walla District. Battelle Pacific Northwest Division, Richland, Washington.

Rakowski, C.L., J.A. Serkowski, M.C. Richmond, and K.P. Recknagle. 2000. Development and Application of a 3D CFD Model for the Bonneville Project Powerhouse 1 and Powerhouse 2. Report submitted to U.S. Army Corps of Engineers, Portland District. Pacific Northwest National Laboratory, Richland, Washington.

Neitzel, D.A., M.C. Richmond, D.D. Dauble, G.Cada. 2000. Laboratory Studies of the Effects of Shear on Fish. U.S. Department of Energy, Idaho Operations, Report DOE/ID-10822.

Perkins, W.A. and M.C. Richmond. 1999. Long-term, One-Dimensional Simulation of Lower Snake River Temperatures for Current and Unimpounded Conditions. Report submitted to U.S. Army Corps of Engineers, Walla Walla District. Pacific Northwest National Laboratory, Richland, Washington.

Richmond, M.C., W.A. Perkins, and T.D. Scheibe. 1999. Two-Dimensional Hydrodynamic, Water Quality, and Fish Exposure Modeling of the Columbia and Snake Rivers. Part 1: Summary and Model Formulation. Report submitted to U.S. Army Corps of Engineers, Walla Walla District. Battelle Pacific Northwest Division, Richland, Washington.

# Mark S. Wigmosta

# Education

Ph.D., University of Washington, Environmental Engineering and Science, 1991

M.S., University of Washington, Geology, 1983

B.S., cum laude, University of Washington, Geology, 1981

Research and professional Experience

1993 - present	Senior Research Engineer II, Pacific Northwest National Laboratories

1991 - 1993	DOE Global Change Distinguished Postdoctoral Fellowship

1989 - 1991	Hydrologist - King Co.  Basin Planning Program

1986 - 1991	Research Assistant, University of Washington

1985 - 1986	Hydrologist and Program Manager Snohomish Co.  Basin Planning Program

1984 - 1985	Hydrologist and Program Manager Snohomish Co.  River Improvement Program

1983 - 1984	Senior Hydrogeologist - Ecology and Environment, Inc.

1980 - 1982	Research Assistant, University of Washington

Selected Publications

Wigmosta, M.S. and L.R. Leung, 2001. Potential impacts of climate change on streamflow and flooding in forested basins, In The influence of Environmental Change on Geomorphological Hazards in Forested Areas, R.C Sidle and M. Chigira, eds., Centre for Agriculture and Biosciences International, in review.

Wigmosta, M.S. and W.A. Perkins, 2000. Simulating the effects of forest roads on watershed hydrology, in The influence of Land Use on the Hydrologic-Geomorphic Responses of Watersheds, M.S. Wigmosta and S.J. Burges, eds., AGU Water Resources Monograph Series, in press.

Leung, L.R., and M.S. Wigmosta, 1999: Potential climate change impacts on mountain watersheds in the Pacific Northwest.  J. Amer. Water Resour. Assoc., 35 (6), 1463-1471.

Wigmosta, M.S., and D.P. Lettenmaier,  1999.  “A Comparison of Simplified Methods for Routing Topographically-Driven Subsurface Flow”, Water Resources Research, 35 (1), 255-264.

Burges, S. J., M.S. Wigmosta, and J. M. Meena, 1998.  “Hydrological Effects of Land-Use Change in a Zero-Order Catchment”, ASCE J. Hydrologic Engineering, 3 (2), p. 86-97.

Wigmosta, M.S., and S.J. Burges,  1997.  “An adaptive modeling and monitoring approach to describe the hydrologic behavior of small catchments”, J. Hydrology, 202 (1-4), p. 48-77.

Wigmosta, M.S., L.R. Leung, and E. Rykiel.  1995.  "Regional Modeling of Climate-Terrestrial Ecosystems Interactions",  J. Biogeography, (22), 453-465.

Wigmosta, M.S., L. W. Vail, and D. P. Lettenmaier.  1994.  “A Distributed Hydrology-Vegetation Model for Complex Terrain.”  Water Resources Research, 30 (6), 1665-1679.

Wigmosta, M.S., and D.P. Lettenmaier.  1994.  "A Distributed Hydrologic Model for Mountainous Catchments", in Advances in Distributed Hydrology, R. Rosso, A. Peano, I. Becchi, and G.A. Bemporad (Editors). Water Resources Publications, Fort Collins, CO, pp. 359-378.

Wigmosta, M.S., and S. J. Burges.  1990.  “Proposed Model for Evaluating Urban Hydrologic Change.”  Journal of Water Resources Planning and Management, Vol. 116, No. 6, pp. 742-763, November/December 1990.

Gregory R. Guensch

### EDUCATION:

M.S.	Watershed Science, Utah State University Fish and Wildlife and Environmental Engineering Departments, 1999

B.S.	Environmental Engineering and Science, University of Florida, 1996

### EXPERIENCE:

8-99-Present	Engineer, Pacific Northwest National Laboratory run by Battelle, Richland, WA

Duties include: Running 1, 2, and 3-dimensional hydraulic and water quality models, quantifying the experimental shear and turbulence exposure to yearling fish, modeling downstream migration of steelhead and chinook smolts

6/99-8/99	Hydrologic Technician, USDA Forest Service, Uinta National Forest, Provo, UT

Duties included: Water quality and nutrient sampling, surveying river geomorphic and habitat features, measuring sediment transport, benthic macroinvertebrate sampling

5/97-5/99	Research Project Leader, Institute of Natural Systems Engineering, Utah State University, Logan, UT

Duties included: Modeling of salmonid position choice, consumption and growth; sampling, sorting and identifying drifting invertebrates; snorkeling and remotely observing fish; electroshocking and stomach pumping fish, and analyzing the gut contents; modeling fish growth with Wisconsin Bioenergetics Model, computing bioenergetic-based habitat suitability metric

4/96-4/97	Research Technician , Geography department, Utah State University, Logan, UT

Duties included: Determining hydrograph characteristics likely to enhance geomorphic features serving as nursery habitat for endangered fish, identifying effects of flow regulation in the Green River, Utah

### EXPERTISE:	Mr. Guensch’s areas of expertise include simulation of the foraging, energy intake and growth of drift-feeding fish; fish habitat modeling; and application of computational hydrodynamic, sediment and contaminant transport, and fluid dynamic models to water quality and fisheries problems.

### PUBLICATIONS, PRESENTATIONS, REPORTS:

Guensch, G., T.B. Hardy, R.C. Addley.  2001. Examining Feeding Strategies and Position Choice of

Drift-feeding Salmonids Using an Individual-based Mechanistic Foraging Model.  Canadian Journal of Fisheries and Aquatic Sciences, 58(3), pp. 446-457

Rakowski, C.L., M.C. Richmond, G.R. Guensch.  2000.  Bonneville Project Adult Migrant Fallback

Study.  Draft Final Report prepared for USACE Portland District by Battelle Pacific Northwest Division.

Guensch, G., Validation of a Mechanistic Habitat Selection Model for Drift-feeding Salmonids.  1999.

Third International Symposium on Ecohydraulics, July 1999.  Salt Lake City, UT.

Guensch, G., 1999.  Validation of a Mechanistic Habitat Selection Model for Drift-feeding Salmonids.

Spring 1999 meeting of the Bonneville Chapter of the American Fisheries Society.  Moab, UT.

##### Scott Waichler

Experience

2000-2001	Senior Research Scientist I, Battelle Pacific Northwest National Laboratory, 				Environmental Technology Division, Richland, WA.

1995-2000	Graduate Research and Teaching Assistant, Oregon State University, Forest Science Laboratory, Corvallis, OR.

1991-1995	Hydrologist, CH2M HILL, Portland, OR.

1989-1991	Graduate Research Assistant, Desert Research Institute, Reno, NV.

Education

Ph.D.	Bioresource Engineering, Oregon State University, 2000.  Statistics Minor.

M.S.	Hydrology/Hydrogeology, University of Nevada, Reno, 1991.

B.S.	Geology, Stanford University, 1984.

Expertise

Areas of expertise include cycling of carbon and nitrogen between soil, plants, and atmosphere, and hydrological and biogeochemical modeling.  Coupled the Distributed Hydrology-Soil-Vegetation Model (DHSVM) and Biome-BioGeochemical Cycles model (Biome-BGC) to address impacts of climate and other environmental change on vegetation and water balance.

Other

Registered Geologist (R.G.), State of Oregon, #G1703.  Member of American Geophysical Union, American Water Resources Association, and American Institute of Hydrology.  Volunteer for The Nature Conservancy on restoration projects at various sites, including Sycan Marsh, Camassia Preserve, and Kingston Prairie, Oregon.

Recent Presentations

Waichler, S.R., 2000, “Simulation of  Vegetation and Hydrology for Climate Change Analysis of a Mountain Watershed”, EOS Transactions, 81(48), F521.

Waichler , S.R., 2000, “Simulation of  Vegetation and Hydrology for Climate Change Analysis of a Mountain Watershed”, PhD thesis, Oregon State University.

Waichler, S.R., 1999, " Variation of Runoff With Vegetation State as Simulated With a

Distributed Hydrology-Biogeochemistry Model," EOS Transactions, 80(46), F351.

Waichler, S.R., B. Jacob, T. Link, 1999, "Supporting an Emerging Academic Water Resources

Community with the WWW,"  Proceedings of the Symposium on Water Resources and the WWW, American Water Resources Association.

Waichler, S., M. Wigmosta, R.P. Neilson, M. Brugnach, 1998, "A hydrology-biogeochemistry model for investigating impacts of climate change and timber harvest on watersheds," EOS Transactions, 79(45), 353.

## GEOFFREY A. MCMICHAEL

EDUCATION

Master of Science, Fish and Wildlife Management, Montana State University, Bozeman.  June 1989.

Bachelor of Science, Biological Sciences, Fish and Wildlife Management option, Montana State University, Bozeman.  June 1987.

EXPERIENCE

1999-present 	Research Scientist, Pacific Northwest National Laboratory

1989 – 1999 	Species Interactions/Ecological Risk Assessment Biologist, Ecological Interactions Team, Washington Department of Fish & Wildlife, Ellensburg, Washington.

1989  			Fishery Biologist, Nature Conservancy, Montana Natural Heritage Program, Helena, Montana.

1987 - 1989 		Graduate Research Assistant, Montana State University, Bozeman, Montana.

# EXPERTISE

Mr. McMichael has been working on laboratory studies of the effects of certain routes of dam passage on juvenile fish. He has been involved in the evaluation of shear stress, turbulence, pressure, and gas supersaturation on juvenile anadromous salmonids. He has also been working on a project to evaluate the spawning habitat selection by endangered chum salmon in the Ives Island area below Bonneville Dam. Geoff's past research emphasis has been in the areas of ecological interactions between hatchery and wild salmonids, behavioral observations, fish population monitoring, predator-prey interactions, and electrofishing injury.

SELECTED PUBLICATIONS

McMichael, G.A., and C.M. Kaya.  1991.  Relations among stream temperature, angling success for rainbow and brown trout, and fisherman satisfaction.  North American Journal of Fisheries Management 11:190-199.

McMichael, G. A., C. S. Sharpe, and T. N. Pearsons.  1997.  Effects of residual hatchery-reared steelhead on growth of wild rainbow trout and spring chinook salmon.  Transactions of the American Fisheries Society 126:230-239.

McMichael, G. A., and T. N. Pearsons.  1998.  Effects of wild juvenile spring chinook salmon on growth and abundance of wild rainbow trout.  Transactions of the American Fisheries Society 127:261-274.

McMichael, G. A., T. N. Pearsons, and S. A. Leider.  1999.  Minimizing ecological impacts of hatchery-reared juvenile steelhead on wild salmonids in a Yakima basin watershed. Pages 365-380 in E. E. Knudsen, and four co-editors, Sustainable fisheries management: Pacific salmon.  CRC Press, Boca Raton, Florida.

McMichael, G. A., T. N. Pearsons, and S. A. Leider.  In Press.  Behavioral interactions among hatchery-reared steelhead smolts and wild Oncorhynchus mykiss in natural streams.  North American Journal of Fisheries Management (Nov. 1999).

McMichael, G. A., T. N. Pearsons, and S. A. Leider.  In prep.  Ecological risks imposed by hatchery steelhead on natural stream salmonid communities. North American Journal of Fisheries Management.

Todd N. Pearsons

EDUCATION

Ph.D.  Fisheries Science.  Oregon State University.  1994.

M.S.  Fisheries Science.  Oregon State University.  1989.

B.A.  Aquatic Biology.  University of California at Santa Barbara.  1985.

RECENT PREVIOUS EMPLOYMENT

RESEARCH SCIENTIST 1, Washington Department of Fish and Wildlife, 7/00-present

ADJUNCT BIOLOGY PROFESSOR, Central Washington University, 3/97-present

FISH AND WILDLIFE BIOLOGIST 4, Washington Department of Fish and Wildlife, 9/95-7/00

FISH AND WILDLIFE BIOLOGIST 3, Washington Department of Fish and Wildlife, 1/92-9/95

CURRENT RESPONSIBILITIES – Director of a research team specializing in ecological interactions and serve as lead ecologist in an interdisciplinary, interagency, research and monitoring team.  Washington Trout recognized the research team with an award of outstanding achievement in research during 1994, and the team was awarded the “best science award” and “best paper award” by the Washington Department of Fish and Wildlife during 1999 and 2000 respectively.

EXPERTISE – Over 17 years of investigating the interrelationships of biota in aquatic ecosystems, 15 of those years involved salmonid ecosystems in the Pacific Northwest.  Co-authored nine BPA technical reports and 17 peer reviewed journal or book publications.  Research has focused on the ecology and monitoring of salmonid ecosystems including food limitation, competition, and predation.  Small-scale experimental approaches to increasing food availability have been evaluated in the John Day and Yakima basins.

SELECTED PUBLICATIONS

Pearsons, T.N., H.W. Li, and G.A. Lamberti.  1992.  Influence of habitat complexity on

Resistance to flooding and resilience of stream fish assemblages.  Transactions of the American Fisheries Society 121:427-436.

Tait, C.K., J.L. Li, G.A. Lamberti, T.N. Pearsons, and H.W. Li.  1994.  Relationships between

riparian cover and the community structure of high desert streams.  Journal of the North American Benthological Society 13:45-56.

Pearsons, T. N., and A. L. Fritts.  1999.  Maximum size of chinook salmon consumed by juvenile

coho salmon.  North American Journal of Fisheries Management 19:165-170.

Pearsons, T. N., and C. W. Hopley.  1999.  A practical approach for assessing ecological risks

associated with fish stocking programs.  Fisheries 24(9):16-23.

Ham, K. D., and T. N. Pearsons.  2000.  Can reduced salmonid population abundance be detected

in time to limit management impacts?  Canadian Journal of Fisheries and Aquatic Sciences 57:17-24.

Ham, K. D., and T. N. Pearsons.  2001.  A practical approach for containing ecological risks

associated with fish stocking programs.  Fisheries 25(4):15-23.