Project ID:	199004402

Title:The Coeur d’Alene Tribe Trout Production Facility

# Section 9 of 10. Project description

a. Abstract

This is an ongoing project sponsored by the Coeur d'Alene Tribe to complement ongoing stream enhancement projects to help protect wild fish stocks by restoring native habitats and improving the function of key watershed processes (NWPPC Program Measures 10.8B.20) and supplementing wild populations with artificial production.  Overall objectives are to increase production of westslope cutthroat trout in four target watersheds by removing and mitigating the causes of habitat degradation and supplementing the native population such that the resulting increased capacity for fish production can be realized as quickly as possible.

To meet the need for off-site mitigation for fish losses on the mainstem Columbia River in a manner consistent with the objectives of the Council’s Program the Coeur d'Alene Tribe is proposing that the BPA fund the operation and maintenance of a trout production facility on the Coeur d'Alene Indian Reservation. The Coeur d'Alene Tribe also proposes that the BPA fund a monitoring and evaluation program consistent with the recommendations of the NWPPC and the ISRP reviews of the project during the 3-step process.

The Coeur d'Alene Tribe Trout Production Facility is intended to rear and release westslope cutthroat trout into rivers and streams with the express purpose of increasing the numbers of fish spawning, incubating and rearing in the natural environment.  It will use the modern technology that hatcheries offer to overcome the mortality resulting from habitat degradation in lakes, rivers, and streams after eggs are laid in the gravel.  Supplementation of native fish stocks in conjunction with effective habitat restoration will be the primary means of achieving these biological goals.

Overarching goals for the program include: 1.) Protection, mitigation, and enhancement of Columbia River Basin native resident fish resources. 2.)  Develop, increase, and reintroduce natural spawning populations of westslope cutthroat trout and bull trout into reservation waters.  3.)  Provide both short and long-term harvest opportunities for the reservation community.  4.) Sustain long-term fitness and genetic integrity of targeted fish populations.  5.) Keep ecological and genetic impacts to non-targeted fish populations to a minimum.

Project specific goals for this proposal is: 1.) Collect, rear and spawn native westslope cutthroat trout in the facility, 2.) Incubate, raise and release progeny from collected broodstock, 3.) Conduct baseline fisheries assessments in affected watersheds prior to release of artificially produced fish, 4.) Conduct a monitoring and evaluation program sufficient to determine the effect of the hatchery fish on the wild population and to determine the effectiveness of the supplementation program in reaching the biological and harvest objectives of the Program, 5.) and, provide for short-term harvest opportunities through maintenance and operation of trout ponds located on the Coeur d'Alene Indian Reservation.

b. Technical and/or scientific background

The activities called for under this proposal provide partial mitigation for the extirpation of anadromous fish resources from usual and accustomed harvest areas and Reservation lands.  Supplementation efforts aim to increase the abundance and distribution of target populations through use of artificial production in conjunction with improvements in instream habitats and watershed processes critical to the survival of westslope cutthroat trout. The Coeur d'Alene Tribe Trout Production Facility is intended to rear and release westslope cutthroat trout into rivers and streams with the express purpose of increasing the numbers of fish spawning, incubating and rearing in the natural environment.  It will use the modern technology that hatcheries offer to overcome the mortality resulting from habitat degradation in lakes, rivers, and streams after eggs are laid in the gravel.  Supplementation of native fish stocks in conjunction with effective habitat restoration will be the primary means of achieving these biological goals.

Biological objectives for wild adfluvial cutthroat trout in tributaries of the Coeur d'Alene Reservation include rebuilding adult populations to 75-100 percent of the optimum level.  This will be accomplished by achieving interim biological objectives (25 percent and 50 percent of optimum level) by the dates noted in the following table 1.

Table 1   Biological and harvest objectives for wild adfluvial cutthroat trout in tributaries of the Coeur d'Alene Reservation (from Cd’A master plan Peters et.al., 2000)

| Tributary | Target Levela (percent) | Escapementb Target | Harvest Targetc | Biologicald Objective | Year |
| --- | --- | --- | --- | --- | --- |
| Alder Creek | 25 | 1,708 | 920 | 2,628 | 2007 |
|  | 50 | 3,416 | 1,840 | 5,256 | 2012 |
|  | 75 | 5,123 | 2,759 | 7,882 | 2016 |
|  | 100 | 6,831 | 3,679 | 10,510 | Beyond |
| Benewah Creek | 25 | 2,179 | 1,174 | 3,353 | 2007 |
|  | 50 | 4,357 | 2,347 | 6,704 | 2012 |
|  | 75 | 6,534 | 3,519 | 10,053 | 2016 |
|  | 100 | 8,713 | 4,692 | 13,405 | Beyond |
| Evans Creek | 25 | 984 | 530 | 1,514 | 2007 |
|  | 50 | 1,968 | 1,060 | 3,028 | 2012 |
|  | 75 | 2,951 | 1,589 | 4,540 | 2016 |
|  | 100 | 3,935 | 2,119 | 6,054 | Beyond |
| Lake Creek | 25 | 2,002 | 1,078 | 3,080 | 2007 |
|  | 50 | 4,004 | 2,156 | 6,160 | 2012 |
|  | 75 | 6,006 | 3,234 | 9,240 | 2016 |
|  | 100 | 8,008 | 4,312 | 12,320 | Beyond |

a  	Target level (percent) is defined as percent improvement over current conditions based on escapement target estimates.

b 	Escapement target is defined as the number of adult fish needed to fully seed available spawning habitat, given the following assumptions:

- Spawning is primarily restricted to 2nd order tributaries (CDA Tribe population data, 1994-1998);

- Usable spawning habitat comprises 4.1% of the total stream area in 2nd order tributaries, when averaged across the four target watersheds (CDA Tribe habitat assessment data, 1998);

- Potential spawning gravel was defined according to Magee et al. (1996) as patches of substrate at least 0.25 m2 in area with particles 2-35 mm in diameter;

- Average redd size is 0.15m2 (Magee et al. 1996).

- 1:1.6 male to female spawner ratio (IDFG 1998);

- 3 redds for every 2 spawning females (Scott and Crossman 1973);

c 	Harvest target is calculated as an exploitation rate of 35 percent.

d	Biological objective is the sum of escapement and harvest targets.

The total number of fish needed to fully seed available spawning habitat was calculated using the following equations:

# of Reds

= Total area of 2nd order streams  *   0.04           Ave. redd size (0.15 m2)

1.5  =                                              1.6 =

# of Reds

# of Females

# of Males

# of Females

# of Males

+                                 = Estimated total number of adult fish to fully seed available spawning area.

Overarching goals for the program include: 1.) Protection, mitigation, and enhancement of Columbia River Basin native resident fish resources. 2.)  Develop, increase, and/or reintroduce natural spawning populations of westslope cutthroat trout into reservation waters.  3.) Provide both short and long-term harvest opportunities for the reservation community.  4.) Sustain long-term fitness and genetic integrity of targeted fish populations.  5.) Keep ecological and genetic impacts to non-targeted fish populations to a minimum.

The Coeur d’Alene Indian Reservation encompasses 139,005 hectares (343,478 acres) in the panhandle section of north Idaho.  Lake and stream resources form the heart of the Reservation and have always been an integral part of Tribal culture.  Tribal peoples utilized abundant anadromous and resident fisheries within their aboriginal territory until the advent of large-scale land use changes by settlers and the construction of dams in the Columbia River system.  Since the extirpation of anadromous fisheries in the upper mainstem Columbia River and Spokane River basins, the Coeur d’Alene Tribe has relied solely on the resident fish resources of Coeur d’Alene Lake and its tributaries to supplement their traditional subsistence diet.

Historically, westslope cutthroat trout were the dominant salmonid in Coeur d'Alene Lake  and the surrounding streams of the Coeur d’Alene basin (Scholz et. al. 1985, Behnke and Wallace 1986).  Historic harvest by Tribal fishermen has been estimated at 42,000 trout annually (Scholz et. al. 1985).  This number decreased to 3,300 fish in 1967, a reduction of 92 percent (Mallet 1968).

Recent efforts to document changes in the distribution of westslope cutthroat trout show significant reductions within Reservation watersheds when compared with the known historic range at various landscape scales (See Table 2).  Furthermore, as the result of persistent, adverse conditions, the average spawning escapements from target watersheds are thought to fall between the minimum viable population and the number of adults needed to produce 50 percent of the carrying capacity of stream environments (Lillengreen, Vitale et al. 1999).  To illustrate the extent of damage, the probability of persistence was calculated for several Reservation populations (See Table 3) using the methods described by Reiman and McIntyre (1993).

| Table 2. Historical and occupied range for westslope cutthroat trout (various sources). Historical range occupied (%) | Occupied range classed as strong (%) | Assessment Area | Source |
| --- | --- | --- | --- |
| 65 | 0 | CDA Reservation | CDA Tribe (Unpublished data) |
| 82 | 11 | Idaho | Reiman/Apperson (1989) |
| 85 | 25 | Interior Columbia Basin | ICBEMP (USFS/BLM) |

Table 3.  Mean annual population estimates, the estimated mean annual variance in the infinitesimal rate of population growth, and probabilities of persistence over 100 years for westslope cutthroat trout populations monitored on the Coeur d’Alene Reservation.  The 95% confidence interval is shown in parentheses (From: Vitale et al. 1999).

| Stream | Years 1996-98 | Mean Annual Population Estimate | Variance | Probability Of Persistence |
| --- | --- | --- | --- | --- |
| Alder Creek | 3 | 808 | 0.03 (0.02-0.04) | 0.58 |
| Benewah Creek | 3 | 5,553 | 0.16 (0.04-0.36) | 0.67 |
| Evans Creek | 3 | 2,675 | 0.33 (0.05-0.71) | 0.45 |
| Lake Creek | 3 | 4,946 | 0.14 (0.02-0.26) | 0.70 |

Despite the apparent instability of cutthroat trout populations in lower elevation tributaries to Coeur d’Alene Lake, preliminary genetic analyses of 16 populations show that relatively pure stocks exist in Reservation waters (Spruell et al. 1999).  Only minimal amounts of hybridization with rainbow trout (Oncorhynchus mykiss) have occurred and some populations show no hybridization at all.  For these same populations, allelic distributions, estimators of pair-wise divergence, and significance measures indicate little correlation between geographic distance and genetic differentiation (Knudsen and Spruell 1999).  Based on an island model of migration, the estimated rate of gene flow among populations is approximately seven individuals per generation (Allendorf & Phelps 1981).  However, this estimate is based on past conditions.  The current level of migration may be reduced since the number of migrants decreases in proportion to the reduction in population size.  Nevertheless, sufficient migration to prevent the loss of rare alleles has probably taken place in the recent past.

The level of genetic differentiation estimated in Coeur d’Alene cutthroat trout by microsatellites appears to be considerably less than estimates from other areas obtained using allozymes.  For example, across the range of the species, the estimated FST is 0.333 (R. F. Leary, pers. comm.).  Within the South Fork of the Flathead River, FST was estimated to be 0.150 (R. F. Leary, pers. comm.).  Both of these values were based on allozymes in which genetic distinction should arise more slowly.  Thus, the microsatellite-based FST estimates presented by Knudsen and Spruell (1999) appear to be quite low for westslope cutthroat trout.  However, levels of heterozygosity appear to be reasonably high, minimizing the possibility that inbreeding depression is currently a problem.

Based on the results of genetic research initiated by this project, it could be theorized that even though westslope cutthroat trout populations are not “strong”, they are not threatened to a large extent with hybridization.  Implications are that if the effect of habitat limiting factors can be reduced, then genetically pure populations would have a chance to recover.

The target watersheds in this proposal have a combined basin area of 34,853 hectares (86,123 acres) and include 529 kilometers (328 miles) of perennial and intermittent stream channel (See Table 4).  It appears that there are a number of factors that contributed to the decline of resident fish stocks within Coeur d'Alene Lake and the tributaries targeted by this proposal.  These factors include construction of Post Falls Dam in 1906, conversion of forested areas to agricultural and pasture lands, increased density of unimproved forest roads, loss of wetlands, and introduction of exotic fish species (Ellis 1932; Oien 1957; Mallet 1969; Scholz et. al. 1985, Lillengreen et. al. 1994, 1996; Peters et al. 1999).  Fragmented ownership presents a particular challenge to restoration efforts.  Tribal ownership represents less than 10 percent of the combined watershed area and private lots are generally less than 80 acres.  Private lands are heavily managed with land use consisting primarily of managed forest (53%), agriculture (25%), and grazed pasture (22%).  For the most part, these agricultural and pasturelands represent lands that were converted from forest and wetland cover types.

Table 4.  Basin morphometry of the Lake Creek, Benewah Creek, Alder Creek and Evans Creek watersheds (From: Lillengreen et al. 1998).

| Characteristic | Lake Creek | Benewah Creek | Alder Creek |  | Evans Creek |
| --- | --- | --- | --- | --- | --- |
| Basin Area | 36.1 mi2 (23,117 ac.) | 58.5 mi2 (37,447 ac.) | 26.6 mi2 (17,047 ac.) |  | 13.3 mi2 (8,512 ac) |
| Basin Length | 10.1 mi. | 13.8 mi. |  | 12.6 mi. | 6.5 mi. |
| Basin Relief | 3,077 ft. | 2,534 ft. | 2,690 ft. |  | 3,278 ft. |
| Basin Perimeter | 34.3 mi. | 51.2 mi. | 26.9 mi. |  | 17.3 mi. |
| Relief Ratio | 0.057 | 0.034 | 0.040 |  | 0.095 |
| Channel Length* | 95.0 mi. | 136.5 mi. | 68.4 mi. |  | 29.0 mi. |
| Drainage Density | 2.51 mi/mi2 | 2.33 mi/mi2 | 2.54 mi/mi2 |  | 2.18 mi/mi2 |

*Includes intermittent tributaries

On private lands, agricultural and forest practices that failed to consider water resources have contributed to the degradation of water quality and habitat suitability for resident salmonids through modification of hydrology, increased sediment transport and delivery, and alteration of riparian plant communities (Kootenai-Shoshone SCD 1991; Peters et al. 1999; CDA Tribe 1998).  Demand for agricultural production and timber products increased sharply following the homesteading of the Reservation, beginning in 1910.  In the Benewah valley, three splash dams and a flume were built in 1915 to float logs out of the watershed.  An estimated 50 MMBF of logs were harvested, principally from riparian areas, between 1915 and 1929 (Hodgson 1998).  Rail lines were developed in both Lake Creek and in Alder Creek to access valuable timber.  The Alder Creek Railroad was built and operated between 1913 and 1927.  Over its fourteen year operational history, some 300 MMBF of logs were sent to market by three or four logging camps with as many as 150 men per camp (St. Maries Gazette Record).  In the late 1930s to mid 1950s the USDA provided incentives for clearing nearly 8,000 acres of forest in the Lake Creek watershed alone (CDA Tribe 1998, 2000).  Consequently, the natural stream flow fluctuations characteristic of these low elevation watersheds was exacerbated (See Figure 1); the target watersheds exhibit extremely low flows in the summer and peak flows have increased by measurable amounts.  In the Lake Creek watershed, for example, peak flows have increased by an estimated 57% and 54% for the 50 and 100 year return intervals, respectively (CDA Tribe 1998).

Figure 1. Water discharge and sampled bedload over time within a second order tributary in the Lake Creek watershed (Boll 2000).

Increased sediment loads from agricultural runoff and unimproved forest roads, and increases in water temperature due to riparian canopy removal may be two of the most critical problems currently affecting tributary populations of westslope cutthroat trout (Peters et.al., 1999). Lake and Benewah creeks are listed on the current 303(d) list revised in 1996 for sediment and temperature violations (USEPA, 1996).  Approximately 97 percent of cropland in the Lake Creek watershed is considered Highly Erodible Land (HEL) under the 1985 Food Security Act, and measured turbidity exceeds water quality criteria of 50 NTU during most significant runoff events (Bauer 1998).  Sheet and rill erosion on these highly erodible agricultural lands, as estimated by the KSSCD (1991) using the Universal Soil Loss Equation (SCS, 1975), was shown to be the primary transport mechanism (See Table 5).  Forest roads, another primary source of sediment, have proliferated in the target watersheds, with densities ranging from 3.4 – 5.7 miles/sq. mile.

Cutthroat trout are sporadically distributed in the Lake Creek, Benewah Creek, and Alder Creek watersheds during both the summer and fall seasons.  Abundance in the second order tributaries of Lake Creek and Benewah Creek are consistently much higher than in adjacent mainstem reaches, despite the effects of low flow conditions.  During base flow conditions, for example, cutthroat trout have been known to crowd into small, isolated pools (>15 fish/m2) located in cool tributaries, rather than face conditions of high water temperatures in mainstem reaches.  In contrast, favorable water quality conditions in Evans Creek result in a relatively even distribution of cutthroat trout.  Cutthroat trout abundance is consistently lowest in Alder Creek.

Alteration and/or conversion of forested riparian habitats have been especially dramatic in the target watersheds.  Agricultural uses, including grazing and cropping, have direct effects on 23%, 36%, 10%, and 60% of riparian habitats in the Alder, Benewah, Evans and Lake Creek watersheds, respectively (Coeur d’Alene Subbasin Summary 2000). General patterns of cutthroat trout abundance and distribution vary among the target watersheds and among years, but seem to be highly correlated to seasonal changes in water quality and quantity.  Measured water temperatures in Alder, Benewah, and Lake creeks regularly exceed 20°C in mainstem reaches during late summer months (Peters et al. 1999; Bauer 1998; Lillengreen et. al. 1996).  These increases in water temperature over time have reduced the range of resident salmonids to a fraction of its historic extent.  Within this new range, sediment has reduced the quality of both spawning and rearing habitats.  Recent population estimates for westslope cutthroat trout indicate that more than 80 percent of available habitat supports less than 2.8 fish/100 sq. meters, while the remaining headwater areas support populations that range from 2 to 15 times greater (Peters et al. 1999).  These headwater “strongholds” illustrate the resiliency of native fish populations and represent the potential for habitat and species recovery in the target watersheds.

Table 5.  Sediment budget summary for the Lake Creek watershed (From: CDA Tribe 1998).

|  | ###### Source |  | Amount generated (tons/year) |  | Delivery to streams (tons/year) |  | Percent of total delivery |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Soil creep |  | 1050 |  | 1050 |  | 7.7 |  |
|  | Mass wasting |  | 14000 |  | 840 |  | 6.2 |  |
|  | Road system |  | 5600 |  | 336 |  | 2.5 |  |
|  | Cropland sheet and rill erosion |  | 115070 |  | 10615 |  | 78 |  |
|  | Cropland gully erosion |  | 12100 |  | 726 |  | 5.4 |  |
|  | Export |  |  |  |  |  |  |  |
| Sediment yield |  | NA |  | 1500** |  | 11 |  |  |

**Estimated export by Lake Creek, 11% of combined inputs

To address these problems, restoration efforts are necessarily targeting the most degraded lands under private ownership, and treating source areas as high priorities.

- Based on the analysis of limiting factors, gains from habitat restoration and protection alone will not achieve the goals set forth by the Coeur d’Alene Tribal Council to provide for self-sustaining populations and harvestable numbers of salmonids, in particular, westslope cutthroat trout.  Thus, the Coeur d'Alene Tribe Fish, Water, and Wildlife Program has proposed and is in the process of constructing a trout production facility to supplement native fish stocks in tributaries located on the Reservation, as well as, provide fish for an interim fishery in trout ponds.  These ponds, located around the Reservation, will provide harvest opportunities to the Reservation community while relieving fishing pressure on weak native salmonid fish stocks while restoration efforts take effect.  Ponds were constructed in isolated watersheds to eliminate the possibility of impacting any native fish populations.  This proposal represents the needs for ongoing operations and maintenance of the facility as well as off reservation baseline fish habitat information needed for monitoring and evaluation of the future supplemented fish stocks.

c. Rationale and significance to Regional Programs

Past mitigation within the Coeur d’Alene subbasin has occurred primarily through implementation efforts of the Coeur d’Alene Tribe as off-site protection, mitigation, enhancement and compensation activities called for under Section 4(h) of the Pacific Northwest Electric Power Planning and Conservation Act and the Northwest Power Planning Council Fish and Wildlife Program.  The activities called for under this proposal provide partial mitigation for the extirpation of anadromous fish resources from usual and accustomed harvest areas and Reservation lands.  Additional mitigation is also occurring to address impacts to resident fish and wildlife populations and habitats attributable to development of the Federal Columbia River Power System (Goal 1, Objective 1 and Goal 2, Objective 1: Coeur d’Alene Subbasin Summary 2000). The relationship of all related projects to the subbasin summary and each other is presented in Table 1.

This project shares the FWP objectives of: maintaining biological diversity in the Upper Columbia River basin; maintaining genetic integrity by preserving wild fish stocks; providing needed habitat protection; and increasing run sizes and resident fish populations by implementing effective restoration projects.  The priorities and objectives of this proposal are described in the 1995 Fish and Wildlife Program [10.8B.20].

Within the Coeur d’Alene subbasin, Coeur d’Alene Lake serves as the principle focal area for the interaction of migratory, resident fishes and the dispersal of genetic material.  It is widely accepted that the persistence of species is linked to the resilience of local populations as well as to the condition, structure, and interaction of populations and habitats at larger scales (Soule 1987; Rieman and McIntyre 1993). The efforts of this project have helped illustrate that migration of individual westslope cutthroat trout over time has prevented the loss of rare alleles in low elevation populations (Knudsen and Spruell 1999; Spruell et al. 1999).  Furthermore, preservation and recovery of the genetically pure populations found in the tributaries addressed by this proposal serves an important role in preserving the genetic integrity of native fishes in the subbasin as a whole (Goal 3, Objective 2: Coeur d’Alene Subbasin Summary 2000).

Habitat degradation caused by overgrazing, road construction, timber harvest and agriculture has adversely affected instream habitat and riparian and floodplain function in streams on the Coeur d’Alene Indian Reservation and throughout the Coeur d’Alene subbasin.  Low summer stream flows, high water temperatures, poor bank stability, sedimentation and a lack of instream and riparian habitat diversity limits trout production and rearing capacity in all but the upper tributaries within the target watersheds (Coeur d’Alene Subbasin Summary 2000; Lillengreen et. al. 1996; Peters et al. 1999).  Despite these challenges, wild fish stocks still persist in each of the target watersheds and demonstrate an ability to respond positively to restoration projects.

# Figure 1. Logic Path for BPA Project 199004402  Coeur d'Alene Tribe Trout Production Facility and for related projects in the Coeur d’Alene subbasin.

Projects called for under this proposal and the related 199004400 are prioritized for implementation in the most critical habitat areas using widely accepted techniques.  Riparian and upland plantings are carefully selected from local, native propagates to maintain and increase diversity.  Four years of intensive implementation efforts has resulted in cooperative relationships with more than twenty-five landowners and eight partnership agencies.  Much work still needs to be accomplished.  With continued funding we anticipate: protecting and enhancing one to two miles of stream channel each year; reducing sediment transport and delivery to streams from highly erosive upland areas; increasing infiltration of water in riparian wetlands; and providing continued protection, maintenance and monitoring of existing projects.

Planning for project implementation is coordinated on a watershed basis that includes the participation and involvement of private landowners, state and federal agencies, and other Tribal programs.  Significant progress was made in recent years to integrate stringent protection measures for riparian dependent resources into management plans adopted by the Coeur d’Alene Tribal Council.  Additional efforts detailed in our program call for development of consolidated memoranda of agreement to address forest road standards and guidelines and schedules for road closure and obliteration on Tribal and privately owned forestlands within the Reservation boundaries.  Furthermore, this project serves as a model for adaptive management through implementation based on watershed assessment and limiting factor analysis, sharing data and objectives with partnership agencies and private landowners, and incorporating effectiveness and trend monitoring to evaluate project responses.

The recent ESA listing of bull trout in June, 1998 and the petition to list westslope cutthroat trout across its range has led to increased recognition that habitat degradation must be reversed to promote species recovery efforts (IDFG 1996; State of Idaho Bull Trout Conservation Plan 1996; USFWS 1998; USDA 1996).  Because private ownership comprises more than 90 percent of the area within the target watersheds, enhancement efforts must necessarily target these lands.  Existing federal programs sponsored by the Farm Service Agency and Natural Resource Conservation Service complement the efforts of this project by providing cost-share monies to implement Best Management Practices on agricultural lands.  Other grants administered by the Tribe (e.g., EPA 319 and 106 grants) leverage additional funding to increase the scope of this project.  The Coeur d’Alene Tribe Fisheries Enhancement Projects have become a logical and integral part of species recovery in the subbasin by implementing projects that establish long term riparian and instream habitat protection on both private and Tribal lands within the Reservation.

d. Relationships to other projects

### BPA Funded Projects

- Project # 199004400 – Implement Fisheries Enhancement Opportunities on the

Coeur d'Alene Reservation

This is an ongoing resident fish substitution project funded through the Bonneville Power Administration to mitigate for lost anadromous fishing opportunities resulting from the construction and operation of Grand Coulee Dam.  Activity associated with the project began in 1987, when the NPPC amended the Columbia River Basin Fish and Wildlife Program to include baseline stream surveys of tributaries located on the Coeur d'Alene Indian Reservation [section 903 (g)(1)(B)].  Initial work used a modified Missouri method (Fajen and Wehnes 1981) to rank reservation streams according to their potential for habitat development for westslope cutthroat trout and bull trout.  Four streams (Alder, Benewah, Evans, Lake Creeks) were identified as having the best potential for restoration and targeted for further study.  Ongoing data collection efforts in combination with baseline information help to promote an adaptive management strategy (as described in section 2.2H of the NPPC F&W Program) in this project.  Habitat and population information collected during the initial baseline investigations was used to produce a limiting factor analysis, which helps prioritize restoration activities.

- Project # 199004401 – Lake Creek Land Acquisition and Enhancement

This project is part of an ongoing effort by the Coeur d'Alene Tribe and the Bonneville Power Administration to protect, enhance, and maintain high value fish and wildlife habitat in the Lake Creek Watershed.  The project involves the enhancement and long-term operation and maintenance of approximately 70 acres of emergent wetlands at the mouth of Lake Creek and 180 acres of associated forested/riparian wetlands.  This area is one component of a recent 2100 acre acquisition that was funded by the Bonneville Power Administration to partially mitigate for resident fish and wildlife losses attributed to the Grand Coulee and Albeni falls hydroelectric facilities.  All activities on the project site complement ongoing habitat restoration work in the Lake Creek Watershed and help to establish a precedent for watershed management efforts on the Reservation.  The enhancement and protection of wetland, riparian, and upland areas will provide measurable improvements in channel stability, sediment abatement, water quality, habitat availability, and suitability for wildlife and fish.

- Project # 9206100 – Albeni Falls Wildlife Mitigation Project

Albeni Falls Wildlife Mitigation Project is a cooperative, multi-agency wildlife mitigation project developed and coordinated through the Albeni Falls Interagency Workgroup.  Implementation efforts are focused at providing in-kind mitigation for habitat types impacted through the construction and inundation of Albeni Falls Dam.  Target habitats identified for protection, restoration, and enhancement efforts include riparian corridors, wetlands, floodplains and scrub-shrub habitats.  Mitigation parcels are currently being pursued on the Coeur d'Alene Reservation through the Albeni Falls Wildlife Mitigation Project.  This project will continue these efforts to develop functional native fish and wildlife habitats on the Coeur d’Alene Indian Reservation.

### EPA Funded Projects

- EPA provides funding to the Coeur d'Alene Tribe Water Resource Program to monitor surface water quality of streams, conduct watershed assessments, and provide the frame work for Total Maximum Daily Load (TMDL) development on the Reservation under section 106 of the Clean Water Act.  Monitoring is conducted at 25 sites within the Reservation including 19 sites within the Coeur d’Alene Lake Basin.  Data is compiled and used to supplement the analysis of fish and macroinvertebrate abundance and distribution on the Reservation.  Data on temperature, discharge and turbidity, in particular, are important in evaluating restoration project effectiveness and coupled with landscape analysis and GIS modeling, provide the necessary information to formulate watershed assessments.  Annual funding for this grant was renewed in FY2000 and should provide approximately $50,000 per year with annual renewal.

- EPA provides funding to the Coeur d'Alene Tribe Water Resource Program for implementing projects for non-point source pollution control under section 319 of the Clean Water Act.  Examples of projects that can be funded include most BMP’s for agriculture, forestry (particularly roads), riparian restoration, and wetland restoration and creation.  Contract awards have ranged from $100,000 to $164,000 during the past funding cycles.

USDA Funded Projects

- The USDA awarded a $360,000 research grant to scientists at the University of Idaho and Washington State University to study watershed restoration on the Coeur d’Alene Indian Reservation.  Scientists on the research team represent the physical sciences, rural sociology, agricultural economy, ecology and conflict resolution.  The grant will provide indicators of restoration success through stream sampling, cost-benefit analysis, and evaluation of landowner attitudes toward erosion control and water quality protection.  The study will continue through September 2001.

- A current project designed to study and develop the watershed restoration process in Lake Creek has been implemented with funding from the USDA (Boll et al., 2000).  Related studies will address a key habitat issue that has not been fully addressed in the past within the Lake Creek watershed, namely, the restoration of salmonid spawning habitat impacted by excess fine sediment at the reach scale.  To accomplish this, the following objectives have been identified and are being addressed: (1) determine the feasibility of natural flows to perform sediment maintenance, and (2) determine what reductions in fine sediments from observed are required to increase embryo survival to an acceptable level.

### Coeur d’Alene Tribe Funded Projects

- The Tribe is initiating the development of an Integrated Resources Management Plan (IRMP) with funding assistance from the USEPA General Assistance Program, USDI Bureau of Indian Affairs, Administration for Native Americans, and Department of Health and Human Services.  The Tribe’s natural resource department is taking the lead on the project, with participation coming from all Tribal programs and departments.  Goals of the project are to create a common vision for the future use and sustainability of Tribal natural, environmental and cultural resources; provide clear goals and objectives to reach the Tribe’s common vision; and coordinate the management of tribal natural, environmental and cultural resources.  Direct benefit will come to the BPA project: Implement Fisheries Enhancement Opportunities through adoption of standards and guidelines for the protection of fish and wildlife resources, and through identification and remediation of conflicting management practices.  The IRMP is the first Tribal management plan that will encompass all natural resources and environmental elements on the Reservation.

- The Coeur d’Alene Tribe Education Department funds a summer internship program for high school students interested in natural resources management.  The Fisheries Program supervises 3-5 of these interns each year and benefits by using them in tree planting and fence building projects, among other things.  Several graduates of the internship program have continued working on restoration projects as seasonal technicians.  It is expected that some of upcoming new interns, as well as employees who started as interns, will work on aspects of projects associated with this Proposal.

- The Names – Stories Project aims to produce GIS maps accessible via computer that are labeled with place names in the Coeur d’Alene language, which are usually derived from a feature of the landscape.  Map locations will be linked to personal stories from Tribal elders about the areas that they describe.  These stories often incorporate an interaction with some landscape or biologic feature and can offer insights into historical conditions.

### Kootenai/Shoshone Soil Conservation District

- The Kootenai/Shoshone Soil Conservation district received a $900,000 grant in 1991 under the State Agricultural Water Quality Program (SAQWP) to be used over a 14 year period.  The grant is used to implement best management practices in the upper Lake Creek watershed for reduction of non-point source non-irrigated cropland erosion.  A considerable amount of momentum has been generated to promote restoration work through integration of this grant with BPA funding.  Cost sharing, as well as sharing of technical expertise, has led to implementation of wetlands enhancement projects and planting of trees to restore native upland forest on formerly cropped land.

### Idaho Department of Fish and Game

- The Idaho Department of Fish and Game has managed native and introduced fish species primarily through the use of fishing regulations.  Restrictive regulations were first used to limit harvest of native cutthroat trout in the St. Joe River drainage in 1970.  Basin wide cutthroat regulations were adopted in 1988.  In 2000, cutthroat regulations in the Spokane River drainage were modified to be more restrictive while reducing regulation complexity.

- The Idaho Department of Fish and Game monitor’s cutthroat and bull trout abundance with both snorkeling and electrofishing surveys in the Coeur d'Alene and St. Joe River drainages.  Snorkeling trend transects have been surveyed since 1970 and electrofishing transects have been surveyed since 1995.  Bull trout redd counts in the Upper St. Joe have been conducted since 1992.

- The Idaho Department of Fish and Game historically stocked rainbow trout over much of the watershed to enhance sport-fishing opportunity.  Stocking of non-native trout has been greatly reduced and is now limited to a total of 28,000 catchable rainbow trout in specific reaches of river.  Since 1998, all of these fish have been sterile triploid rainbow trout produced by heat shocking the eggs.  Stocking is being shifted to catch-out ponds built near traditional stream fishing areas.  The Department has been working primarily with the U.S. Forest Service to build new ponds along the Coeur d'Alene River.

e. Project history (for ongoing projects)

The initial measures for establishing a Coeur d’Alene fish production facility for native trout were amended into the Northwest Power Planning Council's (Council) in 1987.  First steps in this process included a baseline stream survey of tributaries located on the Coeur d'Alene Indian Reservation [section 903 (g)(1)(B)].

In 1994, the Council adopted the recommendations of the Coeur d'Alene Tribe to improve the reservation fishery that were based on the baseline stream surveys.  These recommendations included: 1.) Implement habitat restoration and enhancement measures in Lake, Benewah, Evans, and Alder creeks; 2.) Purchase critical watershed areas for protection of fisheries habitat; 3.) Conduct an educational/outreach program for the general public within the Coeur d'Alene Indian Reservation to facilitate a “holistic” watershed protection process; 4.) Develop an interim fishery for tribal and non-tribal members of the reservation through construction, operation and maintenance of five trout ponds; 5.) Design, construct, operate and maintain a trout production facility, and 6.)  Implement a five-year monitoring program to evaluate the effectiveness of the hatchery and habitat improvement projects (see Program Sections: 10.8B; 10.8B.1; and 10.8B.20).  The master plan (Peters et.al., 2000) submitted to the Council by the Tribe addresses all of these recommendations to some extent, but focuses especially on recommendations 4, 5 and 6.

In July 1997, Congress directed the Council to conduct a review of all federally funded fish hatchery programs in the Columbia River Basin. As this Artificial Production Review (APR) for the Columbia River Basin proceeded, the Council called for artificial production initiatives to be reviewed at three significant stages in development – conceptual design (Step 1, conceptual planning and development of a master plan and approval), preliminary design (Step 2, preliminary design and cost estimation, as well as environmental [NEPA and ESA] review), and final design (Step 3, final design review prior to construction).  This interim process was developed to respond to concerns of the Independent Scientific Review Panel regarding planning and implementation of artificial production projects as well as other Council concerns.  This project is following this process.

The Council adopted the APR report (Document 99-15) at its October 13, 1999 meeting.  This report contains a set of policies intended to guide the use of artificial production in the future as well as recommendations for how to implement the policy reform.  The Council intends to continue to use the three-step process for review of proposed new artificial production programs while incorporating the APR recommendations, as appropriate.  The Council believes that the substantive elements of that review process have been improved by the recently adopted APR report, and that the APR reflects the best available science on this topic.  As such, the Coeur d'Alene Tribe Trout Production Facility has taken into account the contribution of the APR report in the current process.

On November 15, 1999 the Coeur d'Alene Tribe submitted to Council a master plan, as the first step in the three-step review process.  The role of the Coeur d'Alene Trout Production Facility is mitigation for the loss of anadromous fish harvest as a result of elimination/blockage of salmon habitat through construction and operation of Grand Coulee Dam.  Additionally, given the extent of habitat loss from the encroachment of man into the riparian and adjacent lands of tributaries on the Coeur d'Alene Indian Reservation, it is unlikely that natural production in a recovered ecosystem would support all tribal subsistence, and sports harvest interests.

Planning to date has cost $299,490 and includes master plan completion and submittal, conceptual engineering designs and costing, partial staffing to complete necessary work for the submission of the master plan and to provide appropriate training for future hatchery personnel, and genetic analysis.  Earlier efforts that assisted in the development of this project were covered in several projects these cost are not reflected in the above cost.  Additional planning expenses include costs for compliance with National Environmental Policy Act (NEPA), staffing costs, planning costs associated with Step 2 and 3 for preliminary and final designs, and construction management.  The amount currently budgeted for these activities are $651,781 through September 2000.  However, the estimated total cost of completion for this phase of the project will be $932,221 and will take until the Summer of 2001.

Construction of the Coeur d'Alene Trout Production Facility is estimated to cost $3,545,000.  This includes the following items, construction of the facility, construction of four acclimation sites, construction of expansion of the well field to meet water needs and a 10 percent design and a 10 percent construction contingency fund.  This cost does not reflect land acquisition costs for the acclimation sites or for the well field expansion sites or easement costs related to water delivery system.  Land acquisition and easement costs are estimated to be about $64,000.  Annual operation and maintenance costs after all facilities are fully developed would cost about $401,000, excluding additional start up costs.  Monitoring and evaluation is estimated to cost about $300,000 annually.

The Coeur d'Alene Tribe stresses the need to restore the natural functions of the Coeur d'Alene subbasin ecosystem that produce salmonid fishes, as opposed to circumventing natural ecological processes.  The Coeur d'Alene Tribal Fish, Water and Wildlife programs are based on watershed management that equally protects and enhances fish and wildlife resources throughout the Reservation.  Adfluvial cutthroat trout are the target species for supplementation. They are species of special concern throughout the region. This program will participate in as well as help to develop any recovery plans for ESA listed species located on the Reservation or affected by activities conducted.

The proposed project will address partial mitigation (out-of-place, out-of-kind) for anadromous fish losses in the Upper Columbia River basin through a resident fish substitution program. The Coeur d'Alene Tribe Trout Production Facility construction project is one of many ongoing efforts directed at mitigating losses attributed to construction of Grand Coulee and Chief Joseph Dams.

The artificial production program will be designed to produce 10,000 catchable sized rainbow trout for the five catch out ponds and up to 100,000 fingerling cutthroat trout for restoration efforts in the target tributaries (i.e. Alder, Benewah, Evans and Lake creeks).  The hatchery will also support up to 2000 adult cutthroat trout for use as broodstock (Table 1).

Table 1.	Maximum Production Capacity of Coeur d'Alene Trout Production Facility (reprinted from Cd’A Master Plan  Peters et.al., 2000)

| Number of Fish | Ave. Size/ Ave Weight | Species/Life Stage | Pounds Produced Annually |
| --- | --- | --- | --- |
| 2,000 | 20 inch/2.8 lbs. | CTT/ Broodstock | 5,600 lbs. |
| 100,000 | 4 inch/22.6 lbs. per 1000 | CTT/ Fingerling | 2,260 lbs. |
| 10,000 | 12.5 inch/780 lbs. per 1000 | Rainbow Trout/ Adult | 7,800 lbs. |
| Total 122,000 |  |  | 14,660 lbs. |

Broodstock will be collected from each of the four target tributaries.  These fish will be collected as migrating juveniles and held until adults in order to minimize affects on the natural populations.  Each year, initially, 100-200 juveniles will be collected from the same sites in the target watersheds.  These fish will be individually marked and placed into separate raceways.  As these fish mature they will be used as broodstock.

Westslope cutthroat trout will be initially stocked as juveniles.  They will be placed in a stream fed acclimation pond adjacent to the individual target tributary and allowed to leave the pond on their own volition.  If adult returns are poor from this stocking strategy then other types of stocking will be experimented with (i.e. fry plant).  Certified eyed rainbow trout eggs will be purchased and raised in the hatchery.  When ready the fish will be outplanted into the five catch out ponds.  As fish are removed from the pond, more will be added with a maximum of up to 2,000 per pond annually.

The monitoring plan as outlined in the master plan will be revised and amended to address the insights gained from the effort.  In the face of scientific uncertainty, monitoring and evaluation provides this insight into the actual result of an action, as well as, explains the outcome in achieving predicted results. The Coeur d’ Alene Tribal Fish, Water and Wildlife project biologists and managers will initiate an artificial production program that will ensure the experimental studies provide reliable results, and that risks associated with uncertainties are minimized.

Expected benefits from the Coeur d'Alene Tribal Trout Facility include:

- Production of up to 10,000 catchable sized rainbow trout for stocking in five Tribal catch out ponds.

- Increases in the current distribution of westslope cutthroat trout on the Reservation and in Coeur d'Alene Lake.

- Using habitat restoration in conjunction with supplementation to increase the abundance of westslope cutthroat trout on the Reservation and in Coeur d'Alene Lake.

- Reduce the risk of extinction for westslope cutthroat trout in the four target tributaries such that there is a 95 percent probability of persistence over 100 years for each target tributary.

- Increase harvest opportunities by Tribal members and the Reservation community.

- Provide over 20,000 angler hours of opportunity with a catch rate of 0.5 fish/hour at the five Tribal catch out ponds annually.

The proposed facility will contain an indoor central incubation and early life stage rearing facility.  This building will also house the main offices, wet and dry analytical laboratory, and interpretive center.  Four outdoor production raceways will be constructed to hold juvenile fish until they are ready to move to the satellite acclimation facilities.  In addition, four raceways will be constructed to hold broodstock from each of the four tributaries.  Four offsite satellite acclimation facilities will also be constructed to hold the fish for acclimation and volitional releases.

Two earthen grow out ponds to support the rainbow trout for the catch out ponds will be constructed.  Rainbow trout will be kept in the grow out ponds for an additional period.  These ponds will serve as a distribution center for the rainbow trout until they are outplanted into the five catch out ponds.  The rainbow trout grow out ponds will be approximately 75 feet wide and 190 feet long each.  Total pond depth will vary from 15 ft at the supply end to 16 ft at the drain end to facilitate drainage

This proposal describes an ongoing mitigation project in its first full year of operation thus, previous results are scant and project history focuses on planning and what we are going to accomplish. However, this project is the culmination of ten years of preliminary work conducted by BPA Project 199004400, entitled Implement Fisheries Enhancement Opportunities on the Coeur d’Alene Indian Reservation.  Project 199004400 has been underway since 1990.  Annual funding for this project has averaged $442,342 and total costs from 1990 through 2000 were $4,423,416.  For the period 1995-2001, this project will supplement regular program funding with more than $900,000, with monies coming from EPA Clean Water Act Grants, an EPA Environmental Education Grant, a USDA research grant, and State Agricultural Water Quality Program funds, among others.  Since 1997, landowner contracts have provided for additional cost shares where private landowners provide long term maintenance of projects.

Previous project reports include: annual and biennial project evaluation reports 1990, 1991, 1992, 1993-94, 1996-97, 1998 (Graves, et. al. 1992; Lillengreen et. al. 1993; 1994; 1996; Vitale et al. 1999); Tribal management plan for enhancement of resident fisheries (Lillengreen et. al. 1998); a supplementation feasibility report (Peters et al. 1999); Trout Production Master Plan (Peters et al. 1999); a stock assessment for westslope cutthroat trout (Peters and Vitale 1999); and two reports on genetic analysis of westslope cutthroat trout in tributaries of Coeur d’Alene Lake (Spruell et al. 1999 and Knudsen and Spruell 1999).  A biennial report for 1999-2000, evaluating restoration projects and monitoring efforts, is currently being prepared.  Information derived from this project has also been disseminated through project presentations given to the American Fisheries Society (Vitale 2000) and Idaho Academy of Sciences (Peters 1999), at conferences on native species and habitat restoration (Vitale 1999; Vitale 1997), and at numerous regional caucuses sponsored by NWPPC, CBFWA and BPA.

Physical and biological surveys incorporated measures of stream bank and bed stability, riparian condition, land use, urbanization, migration barriers, water quality, stream flow, substrate suitability, channel modification, relative abundance estimates, and macroinvertebrate densities (Fejen and Wehnes 1981).  Four streams (Alder, Benewah, Evans, and Lake Creeks) were identified as having the best potential for habitat restoration and species recovery and targeted for further study.

Between 1992 and 1994 watershed assessment techniques were used in the target drainages to identify priority areas for restoration by examining sediment routing (Washington Forest Practice Board 1992; Kappesser 1992), channel function and stability (Rosgen 1994; Pfancuch 1975;), instream habitat composition (Bisson et. al. 1981), riparian function (Platts et. al 1983), as well as the abundance and distribution of target species.  Parameters looked at included: number of pools, riffles, and secondary channels; large organic debris volume; riparian vegetation diversity and stream canopy closure; land use practices in floodplain areas; and water quality parameters.  Measures of substrate composition and percent fines were also used as habitat quality indicators.  Biological assessments included trout population and biomass estimates, individual stock assessments, and quantification of macroinvertebrate populations (Plafkin et. al. 1989).

It was determined that in all four target streams base flow, summer water temperature, and fine sediment are limiting the quality of trout habitat.  Average annual base flows of less than 25% of average annual flows were reported in all streams.  Temperatures above those values reported as optimal were recorded in each of the four streams.  Non-point source sediment runoff is also considered limiting in all four drainages.  Average % fines for all streams were above the recommended 10% fines value.  It was determined that habitat degradation (a result of the cumulative effects of many land use practices), as well as, introduction of exotic salmonid fish species has caused the extinction of bull trout and severely depressed native populations of westslope cutthroat trout.  Land use practices have altered natural ecosystem processes with which native fishes have evolved.  This alteration has created conditions less favorable for native salmonids and more favorable for introduced exotics, as well as, less desirable native fishes.

Demonstration projects were implemented in 1995 and 1996 with the cooperation of private landowners.  A primary objective of these first projects was to evaluate the application of a variety of techniques to specific watershed conditions.  Passive regeneration techniques, using riparian exclosure fencing as the primary method, were subsequently adopted to restore degraded habitat.  This has been proven in other studies to be an effective means of improving riverine/riparian habitats (Beschta et. al. 1991; Platts 1990).  Active remediation techniques using plantings, bioengineered stream bank protection, and placement of instream structures made from native materials have also been used when natural processes have been deemed unlikely to result in recovery within a desired time frame.  To date over 42 individual habitat restoration projects have been completed as a result of this project.  A more detailed project history and statement of results is contained in the project proposal narrative for project # 199004400.

f. Proposal objectives, tasks and methods

## Section 5. Construction/Implementation phase

Objective 1. 	Collect, rear and spawn native westslope cutthroat trout, utilize as                           broodstock.

Task 1a.      Capture/collect emigrating juvenile westslope cutthroat trout.

Methods:   Emigrating juvenile westslope cutthroat trout captured utilizing a modification of the juvenile downstream trap found in Conlin and Tuty (1979).

Task 1b.   Rear captured juveniles for future propagation efforts.

Methods:    Captured emigrating juveniles to be utilized for future propagation efforts are transported via a transport truck to the trout production facility.  Newly recruited juveniles are held in isolation/quarantine for duration of six to nine months to determine their fitness. Fish that are deemed fit are reared/housed in brood ponds until they mature (Peters et. al. 2000).

Task 1c. Spawning of the broodstock.

Methods:   Eggs and milt mix in a stainless steel bowl, which contains a ten-percent saline solution, the solution amount is approximately one ounce per bowl. This saline solution aids in sperm mobility.

Fish are netted into a trough that has approximately one heaping tablespoon full of methane tricanesulfonate (MS-222), one cup of rock salt, and approximately 20 gallons of water. The MS-222 sedates the fish, the salt reduces/aids in stress management, and the water is the solvent and medium for the drugs to be administered to the fish.

First, two to three males are stripped of milt into the saline solution, then females are stripped of their eggs into the saline solution with the milt. During the spawning efforts if approximately three minutes pass during the same batch, more milt is required to ensure fertilization.

After the batch is completed, eggs mixed with the milt are transferred to a common bucket. The common bucket is then rinsed with clean fresh water.  The rinsing is to expel excess milt and sub-par eggs (eggs that aren’t viable).

Freshly rinsed fertilized eggs are combined in a five-gallon bucket. This five-gallon bucket is either filled or partial filled from the days’ spawning efforts. This bucket when filled or partially filled is transported to the hatchery, although during transportation of the fertilized eggs they are covered/shielded with an raincoat to protect them from ultra violet radiation, which is believed to be harmful to reproductive parts. Fertilized eggs are water hardened for approximately one hour.  After water hardening eggs are disinfected with an iodine solution that is adjusted for pH, approximate duration of ten minutes.  After disinfecting the fertilized eggs they are enumerated and placed into incubators following specific numbers from manufactures instructions.

Objective 2. Incubate, rear and release progeny into targeted streams.

Task 2a.   Incubate fertilized eggs in vertical stack type trays (Heath tray type).

Methods:  Incubating fertilized eggs are provided with fresh water supply, with the top tray of each stack utilized for silt collection and a mixing stage fungal treatment, a formalin solution.  The eggs inspected daily to determine the status of their development.  When present, dead or dying eggs (opaque in color) are removed and enumerated for historical use.  Incubating eggs are also shocked by placing the incubating eggs into a bucket and poured from one bucket to another at approximately waist height and filled approximately one third with clean water and the process repeated thee times.  Shocking the eggs has little effect on strong viable eggs, but causes inferior eggs to die.  Shocked eggs after a few days are separated and enumerated with an electric egg-sorting machine.  Fugal treatments are continued daily until the first signs of hatches are observed.

Task2b.  Rear newly hatched progeny

Methods: Newly hatched progeny will be placed in fiberglass troughs in hatchery building where they are trained on feed.  After progeny are trained on artificial feed they will be placed in production raceways.

Task2c.  Release progeny into targeted streams.

Methods: Reared progeny at an approximate size of four inches are transported to acclimation ponds on targeted streams via transportation truck designed to transport live fish.  Fish will be held for volitional release beginning about two months after transport.  Fish will be flushed from the acclimation pond when acclimation pond temperatures raise above 15 C.

### Section 6. Operation and Maintenance phase

Objective 1. Facilities maintenance

Task 1a. Facility Operation and Maintenance

Methods: 	Due to the complexity of the reuse system, staff are to be trained to service minor components of the system and major components serviced by qualified personnel if required.

Task 1b.  	Perform routine maintenance and servicing on ground water (well)  system.

Methods:	Staff are to be trained to service minor components of the system and major components serviced by qualified personnel if required.

Task 1c  	Identify annual facility electric and utilities costs.

Methods:	Annual budgets will be produced estimating costs.

Task 1d.  	Purchase additional water (approximately 20gpm) to supplement ground water supply.

Methods:	The town of Worley, ID will be contracted to supply 20 GPM continuous feed water from their supply to support the facility.  The system will be tied directly into the hatchery water supply.  The system will be capable of supplying up to 60 GPM in the event of an emergency.

Objective 2. 	Perform maintenance at each pond site as a safety measure for the public.

Methods:	Due to high fishing pressure on the rainbow trout ponds, weekly maintenance is required to maintain safety and aesthetics.  Grass will be mowed, trash will be picked up and trashcans will be dumped on a weekly basis.  Docks will be maintained for safety and to allow access for disabled anglers.

Section 7.  Monitoring and Evaluation phase

Objective 1.	Evaluate effectiveness of Rainbow trout stocking program.

Task 1a.	Conduct creel census and public opinion surveys for all ponds in the program to determine angler use and attitudes.

Methods:	A stratified random sampling method and a roving creel survey will be used to survey the public following the methods described by (Malvestuto, 1983).  A standard form will be design to allow for an unbiased creel census to be conducted at all put and take ponds with in the Coeur d'Alene Reservation.  Information to be gathered will include number of fish caught, duration of fishing, if the angler was satisfied or dissatisfied with the size of the fish and aesthetics of the site.

Task 1b.	Calculate population estimates of remaining fish at each pond as a measure of fishing pressure.

# Methods:	Each pond will be seined two times annually, at least one week apart at the end of the season, in order to perform population estimates.  Three pulls will be made at each pond and all fish sampled will be marked with a fin clip to allow for easy identification.  Because the fish will be planted only lengths will be recorded on them to minimize handling time and monitor growth.  A simple proportional population estimate will be made using the percentage of marked fish vs. unmarked fish captured during the second seine.

Task 1c.	Part of our public outreach program is to construct information boards.

Methods:	This will be a project for the summer youth interns hired during the summer months.  They will be instructed to build information boards that will be displayed at each pond site.  The boards will provide information on fishing season, fishing limits and other local information.

Objective 2. 	Evaluate the existing compensatory harvest opportunities on the Reservation.

Task 2a	Review existing management plan and make adaptive changes to ensure that fishing pressure on wild stocks remains at acceptable levels.

Methods:	Results from 1a and 1b will all us to evaluate catch per unit effort (CPUE) and the fishing effort that each of the ponds receives from anglers.  CPUE will allow us to estimate total harvest or harvest by species over a specified time period, obtain an index of stock abundance for particular species, and to measure fishing quality or fishing success (Malvestuto, 1983).  Fishing effort will be based on angler count data.

Objective 3.	Conduct limiting factor analysis for hatchery westslope cutthroat trout in Coeur d'Alene Lake.

Task 3a.	Coordinate with local agencies to gather baseline predator prey relationships between large piscivorous fish and their prey located in Coeur d'Alene Lake.

Methods:	The Coeur d'Alene Tribe Fisheries Program in coordination with the Idaho Department of Fish and Game will gather baseline predator prey relationships in Coeur d'Alene Tribe ceded territories.

Shoreline Electrofishing

Any personnel serving as electrofishing team leaders must demonstrate knowledge of the principles and techniques of electrofishing.  Team leaders will be considered knowledgeable of the principles and techniques of electrofishing upon satisfactory completion of the National Fisheries Academy course, Principles and Techniques of Electrofishing.

Electroshocking, gillnetting and beach seining along with the sampling schedule is designed to capture data related to significant changes in relative abundance of various fish species throughout the year.  A custom built aluminum boat equipped with a Smith Root 7.5 GPP electroshock unit will be used to conduct shoreline electrofishing.  Electroshocking is designed to effectively shock a depth of 0 to 8 feet.  The electrofishing sampling technique is ineffective in waters greater then 8 feet deep.  Each reach within a transect will receive a 5 minute shocking effort depending on fish density.  All captured fish will be placed in an aerated live well until the shocking effort is completed.  Captured fish will be identified, enumerated, measured (TL to nearest mm), and weighed.  Westslope cutthroat trout, bull trout, mountain whitefish, northern pike, largemouth bass, smallmouth bass and chinook salmon will be tagged with a Floy FD-6B numbered anchor tag if they have a length greater than 300mm and weigh greater than 300g.

Gillnet Sampling

Two different types of nets are used for fish sampling in Coeur d'Alene Lake.  Vertical gillnets are used in areas with water depths greater than 30 feet (pelagic zones).  The vertical gillnets that will be used are 6 feet wide and 120 feet in length.  They have various mesh sizes: 2.5”, 3”, 4”, and 5 inches. These nets are used specifically to sample areas that are inefficiently sampled using the electrofishing technique.  Depth of capture, as well as, length, weight and age data is included in the final analysis.  In the shallower areas (8 - 30 feet) horizontal gillnets will be used.  The horizontal gillnets that will be used are graded mesh monofilamant 8’X200’, 10’X200’ and 12’X150’.  The horizontal gillnets will be used to examine the species composition and the seasonallity of the species composition in the littoral and limnetic zones.

Beach Seining

Beach seining will be used to sample young of the year fish that inhabit the shoreline.  Electroshocking and gillnetting methods are ineffective at sampling this age class (Peters et. al, 1999 and Vitale et. al. 1999).  Beach seining will be completed using a 200-foot net that is tapered from 10 feet in the middle to 4 feet on each end.  The seine will be set along the shoreline where water depth will rang from 10 to 12 feet.  Two pulls will be completed at each sampling site.  Sampling will take place during the daylight and nighttime hours.  Fish from the first pull will be set aside in aerated coolers that contain water.  Immediately after the second pull is complete the fish from the first pull will be released back into Coeur d'Alene Lake.  Bins will be set up based on the length of the fish.  Length measurements will be taken on all species.  Length, weight and scales will be taken on the first five of the following species: westslope cutthroat trout, bull trout, mountain whitefish, largescale sucker, northern pikeminnow, northern pike, largemouth bass, smallmouth bass, chinook salmon, kokanee salmon, yellow perch, and black crappie all other fish species will be measured for length and weight only.

Age Analysis

Age will be determined on the species listed above in order to develop age frequency distributions.  The age frequency distribution of a representative population sample describes the status of successive cohorts.  By examining age frequency over time, instances of mortality as well as overall population trends will be identifiable.  Age will be determined using methods described by (Jearld, 1983).

Habitat Suitability Index

Lacustrine habitat suitability index model developed by Hickman and Raleigh (1982) was used to predict suitable habitat for cutthroat trout in Coeur d’Alene Lake.  The model consists of a water quality and a reproduction component.  The lacustrine water quality component uses three variables: maximum temperature (V1), average minimum dissolved oxygen (V3) and pH (V13).  Individual suitability index (SI) values were calculated for each variable using curves published in Hickman and Raleigh (1982).  The following equation was used to ascertain the final SI values:

Cwq = (V1xV3xV13)1/3

Where Cwq = HSI for the water quality component, and

Vn = suitability index for water quality variables.

Monitoring Schedule:

Sampling is initiated just prior to the onset of the growing season and continues until the lake turns over in the fall, which typically marks the end of the growing season.  This season (2000) sampling will begin in April and end in October.  We will try to capture a representative sample during the winter and apply this data to the rest of the winter season.  Fish will be sampled twice along each sample reach each month in April, June, August and October by electroshocking.  One sample will be completed during daylight hours and one sample will be completed during the nighttime hours.  Gillnetting will coincide with the electroshocking schedule.  Beach seining will occur in May, July, September.  Each transect will have one site that will be sampled twice.  See appendix A for the complete sampling schedule for all fish sampling sites.

Data Analyses and Reporting:

Data will be compiled on an electronic data logger and downloaded.  Data summaries will be completed monthly.  In February of 2001, an annual report containing the previous seasons data will be completed.  Prior to the start of the next field season a standard operating procedure for the upcoming season will be completed.  Sample sites, parameters, and schedules will be completed based on information gained from the previous years data.

Objective 3 continued-

Task 3b.	Compare eutrophication in the northern section of Coeur d'Alene Lake to the southern section of Coeur d'Alene Lake by gathering baseline productivity information (nutrients, TKN, TP, turbidity, TSS, metals and chlorophylla).

Methods:

Data from past studies will be used along with the data that the Coeur d'Alene Tribe Fisheries Program collects.  The Coeur d'Alene Tribe Fisheries program will be coordinating collection efforts with local agencies.

Physical:

Each water quality station will be sampled using the hydrolab.  The hydrolab, a trade name for a multiparameter water quality testing probe, will be used to determine dissolved oxygen, pH, temperature, conductivity, and total dissolved solids from a calculation using conductivity measurements.  The unit will be calibrated prior to each use according to manufactures specifications outlined in the operating manual.  A calibration log will be kept in order to verify that calibration procedures were completed.  The calibration procedures and instrument records can be found in the master logbook.  In addition, correct calibration will be verified monthly using scientific grade reference solutions.  At the end of the day instrument drift will be checked.  Results will also be kept in the master logbook.  Once deployed, the transmitter will be allowed to stabilize then one instantaneous reading will be taken and the hydrolab is moved to the next depth.  A datalogger (which is a field computer) will be used to log the data.

Secchi disk is a black and white weighted circular plate 20 cm in diameter that will be used to measure water transparency.  Secchi disk readings are useful in assessing water quality, particularly algae and suspended sediment.  The secchi disk reading determines where the epilimnion water sample will be taken.  The thermocline determines where the hypolimnion water sample will be taken.

Water Quality Collection Methods:

Water samples will be collected using methods described in the Standard Methods for the Treatment of Water and Wastewater (APHA, 1995).  All analyses of this type will be conducted by an accredited (either state or EPA certification) contract laboratory (e.g. Spokane Tribal Laboratory).  Composite water samples will be taken in the epilimnion and hypolimnion at each site.  The epilimnion sample is taken 1m below the surface and 1m below the secchi reading.  The hypolimnion sample is taken 1m above the lake bottom and 1m below the thermocline.

Turbidity:

Turbidity is the amount of suspended inorganic and organic matter throughout the water column.  Turbidity measurements will be determined through use of a HACH Model 2100P portable turbidimeter.  The turbidimeter measures turbidity from 0.01 to 1000 Nephelometric Turbidity Units (NTU).  The unit will be calibrated according to manufacture specifications.  Frequency of calibration will be conducted as required for EPA reporting under NPDES standards.  As such, quarterly calibration of the unit using primary formazin standards will be completed.  Calibrations using Gelex secondary standards will be conducted on a bi-weekly basis.  Samples will be collected simultaneously with the hydrolab data.  The calibration results can be found in the master logbook.  In addition to using the HACH composite water samples collected from the epilimnion and hypolimnion will be sent to a contract laboratory to be analyzed and used to check the accuracy of the HACH.  A contract laboratory using EPA method 180.1 will run duplicate samples for quality control.

Total Suspended Solids:

Composite Total Suspended Solids (TSS) water quality samples will be taken by the methods described in the water quality collection above.  TSS is defined as the residue left on a filter paper of 2m or smaller after a portion of sample has been filtered through (APHA, 1992).  TSS is analyzed using EPA method 160.2: Gravimetric determination of Total Suspended Solids.

Metals:

Composite metal samples will be taken in the epilimnion and hypolimnion as described above in the water quality collection methods.  Metal samples are preserved by acidification to 2% HNO3 as soon as possible after the collection is taken.  Metals will be analyzed using EPA method 200.7 Inductive coupled plasma.  The following trace elements will be analyzed: zinc, silica, antimony, barium, beryllium, magnesium, arsenic, sodium, aluminum, calcium, copper, silver, lead, cadmium, cobalt, nickel, manganese, iron, chromium (Supplementation Feasibility Report, 1998).

Nutrients:

Nutrient water samples will be taken as described in the water collection methods.  Nutrients will be analyzed using EPA method 300.0 Ion chromatography.  Ions sampled are chloride, fluoride, nitrate, nitrite, ortho-phosphate, and sulfate.

Total Kjeldahl Nitrogen:

Total Kjeldahl Nitrogen (TKN) water samples will be taken as described in the water quality collection methods.  Organic nitrogen is defined functionally as organically bound nitrogen in the tri-negative oxidation state.  Analytically, organic nitrogen and ammonia can be determined together and have been referred to as “kjeldahl nitrogen, a term that reflects the technique used in their determination (Standard Methods 18th Edition, 1992).  Total kjeldahl nitrogen will be analyzed using standard method 4500-NorgB by a qualified laboratory.

Total Phosphate:

Total Phosphate (TP) water samples will be taken as described in the water quality collection methods.  Phosphorus is essential to the growth of organisms and can be the nutrient that limits the primary productivity.  Total phosphate measures the amount of orthophosphate, condensed phosphate and organically bound phosphate in the system.  Orthophosphates applied to agricultural or residential cultivated land as fertilizers are carried into surface waters with storm run off and to a lesser extent melting snow.  Small amounts of certain condensed phosphates are added to some water supplies during treatment and laundry detergents may also have condensed phosphates.  Organic phosphates are formed primarily by biological processes (Standard Methods 18th Edition, 1992).  Total phosphate will be analyzed using EPA method 200.7 Inductively Coupled Plasma by a qualified laboratory.

Primary Productivity:

Chlorophylla water samples will be taken as described in the water quality collection methods.  Chlorophylla is the pigment aquatic plants use for photosynthesis.  Chlorophylla samples allow us to measure algae abundance.  Determination of chlorophylla will be by Standard Method 10200.H Spectrophotometric by a contract laboratory.

Monitoring Schedule:

All lake sites will be sampled for hydrolab parameters (dissolved oxygen, temperature, pH and conductivity) on a bi-weekly basis March 1 - October 30 and on a monthly basis November 1 - February 28 except when ice covered.  Secchi disk sampling will be completed concurrently with hydrolab sampling.  Nutrients will be sampled monthly from March 1 - October 30 (eight times) and once November 1 - February 28.

Data Analyses and Reporting:

Data will be compiled on an electronic datalogger and at the end of each sampling period it will be downloaded.  Data summaries will be completed monthly.  In February, an annual report containing the previous years data will be completed.  The annual monitoring report will contain the objectives and goals, standard operating procedures (SOP), quality assurance and quality control (QA/QC) assurance reviews, data tables, summaries, and interpretations.  Prior to the start of the next field season SOP for the upcoming season will be completed.  Sample sites, parameters, and schedules will be completed based on information gained from the previous years data.

Objective 3 continued-

Task 3c.	Monitor phytoplankton, zooplankton, and aquatic macrophytes within Coeur d'Alene Lake.

Methods:

Phytoplankton / Zooplankton:

In order to increase relative abundance of westslope cutthroat trout in Coeur d'Alene Lake there needs to an available food source.  Juvenile cutthroat trout feed on zooplankton along with aquatic invertebrates.  Phytoplankton (qualitative only) and zooplankton will be collected using a turtox plankton net.  The net size is 200mm mouth X 890mm long with an aperture size of 80m.  A 125mL plastic bottle will be placed on the end of the net to collect the samples.  Samples will consist of near shore and limnetic tows. Samples will be preserved using 10% Lugols.  Samples will be sent to a certified laboratory for identification.

Monitoring Schedule:

Samples will be collected at the same time that the physical and chemical water quality data is being gathered on Coeur d'Alene Lake.  Samples will be taken on a bi-monthly basis from March through May.  Monthly samples will be collected from June through November.

Aquatic Macrophytes:

Aquatic macrophytes will be sampled within the established electroshocking and beach seining transects.  Transects will be sampled at 1 meter intervals from depths ranging from 1m to10m.  Two different methods are being considered for the collection of aquatic macrophyte samples.  The first is to collect the samples by scuba diving the transects over a known distance.  The scuba diver will be certified in open water dives.  This might allow for easier collection and less disturbance to the lake bottom.  It would also allow us to sample deeper water.  The second method uses methods described by Loescher et. al., 1996.  They use a steel rake that is dragged behind a boat for a known distance.  In both cases the samples will be placed in bags and labeled.  The samples will be stored until they can be transported to a contracted laboratory for identification.  The samples then will be pressed and kept on file in the Coeur d'Alene Tribe Fisheries office.

Objective 4.	Conduct baseline fish population assessments in low elevation tributaries of Coeur d'Alene Lake to determine effects on wild populations resulting from releases of hatchery produced westslope cutthroat trout.

Task 4a.	Identify fish assemblages in St. Joe River from Mica Creek downstream to the mouth excluding St. Maries River.

Task 4b.	Identify fish assemblages in mainstem Coeur d’Alene River.

Task 4c.	Identify fish assemblages in Coeur d’Alene Lake tributaries.

Methods:

All hatchery westslope cutthroat trout will receive an adipose fin clip before being put in the acclimation ponds.  This method will allow for easy identification from wild stock by anglers and fisheries personnel.  Pit tags are also being examined as a more effective way of monitoring straying and migration by cutthroat trout.  Migration traps will be installed during the spring to survey juvenile and adult fish.

### Fish Population Evaluation

Annual sampling will be conducted at established reference reaches to determine baseline population status, distribution of westslope cutthroat trout, and to measure biological response to enhancement techniques.  Sample sites within each reach will be selected to include habitat types representative of the reach as a whole.  Sampling will be conducted in the summer to quantify the abundance and distribution of fishes during base flow conditions.  We understand that there are many fish population analysis techniques.  We intend to undergo a complete literature review of these techniques prior to initiating sampling.  However, we have had success with the technique described below.  It appears that estimates derived from this technique are partially corroborated with carrying capacity (both individuals and standing crop biomass) estimates described in the Coeur d'Alene Tribe facility master plan.

Trout populations will be estimated using the removal-depletion method (Seber and LeCren 1967, Zippen 1958).  Blocknets will be placed at the upstream and downstream boundaries to prevent immigration and emigration.  Each sample site will be electrofished using the standard guidelines and procedures described by Reynolds (1983) and the National Marine Fisheries Service (1998).  To minimize the negative effects associated with electrofishing, visual counts will be made using snorkeling techniques wherever habitat conditions permit.

Captured fish will be identified, enumerated, measured (TL to nearest mm), and weighed.  Cutthroat trout that have a length greater than 200mm will be tagged with a Floy FD-6B numbered anchor tag.

We will use MicroFish V3.0 to calculate fisheries population statistics.  MicroFish V3.0 is a microcomputer software system designed by Van Deventer and Platts (1987) for calculating fisheries population statistics from electrofishing data.  System output includes such things as maximum-likelihood population estimates, total catches, capture probabilities, removal patterns, lengths, weights, condition factors, and biomass.  Relative percentages, standard errors, and confidence intervals are generated for each sampling site and species in the data set.  Allen et. al (1995, 1997, 1998) used MicroFish V3.0 to calculate population estimates and confidence intervals for redband trout.  We propose to use equations published by Armour et al. (1983) to provide initial verification of the software system.

Age will be determined for all trout species captured in order to develop age frequency distributions.  The age frequency distribution of a representative population sample describes the status of successive cohorts.  By examining age frequency over time, instances of mortality as well as overall population trends will be identifiable.  Age determination will be made using methods described by (Jearld, 1983).

Objective 5.	Initiate baseline limiting factor analysis for naturally produced salmonids found in low elevation tributaries to Coeur d'Alene Lake.

Task 5a.	Identify habitat-limiting factors in both the tributaries of the St. Joe River from Mica Creek downstream to the mouth excluding St. Maries River and the mainstem St. Joe River.

Task 5b.	Identify habitat-limiting factors in and mainstem Coeur d’Alene River.

Task 5c.	Identify habitat-limiting factors in Coeur d’Alene Lake tributaries.

Methods:

If straying occurs from the hatchery cutthroat trout and they pioneer into other low elevation streams then monitoring will take need to take place in these streams to identify production enhancement potential.  Habitat restoration activities may occur in these streams after they have been assessed by the following methods if it was determined feasible to enhance production.

### Channel Classification

Stream pattern morphology is directly influenced by eight major variables including channel width, depth, velocity, discharge, channel slope, roughness of channel materials, sediment load, and sediment size (Leopold et al. 1964).  A change in any one of these variables sets up a series of adjustments in the other variables, resulting in channel pattern alteration (Rosgen 1996).  For example, for many stream reaches within the Hangman Creek watershed manipulation of riparian vegetation has compromised streambank stability.  This has lead to an increasing width/depth ratio; decreased sinuosity; increased slope; increased bar deposition; accelerated bank erosion; increased sediment supply; decreased sediment transport capacity; a decreased meander width ratio; and channel aggradation (Spokane County Water Conservation District 1994).

The use of channel classification as a tool for understanding and predicting channel responses such as these is not new (Kellerhals et al. 1972, 1976; Galay et al. 1973; Mollard 1973; Montgomery and Buffington 1993).  For this project we propose to apply the channel classification developed by Rosgen (1994) as the basis for providing a consistent initial framework for organizing river information and to correlate similar general level inventories that will follow, including: aquatic habitat inventories, fish population evaluations, bioassessment of macroinvertebrate communities, water quality inventories, and development of restoration priorities.

Implementing the hierarchy of inventory or assessment levels described by Rosgen (1996) will provide the Tribe with the most comprehensive tool leading from broad geomorphic characterization to very detailed-specific assessment and finally to implementation, monitoring and evaluation.  Having quantitative data on the variables that influence stream pattern morphology will provide us with a critical tool to restore Reservation streams and their associated riparian and floodplain communities to their full stream potential.  Stream potential has been described in terms of “Desired Future Condition” (USDA Forest Service 1992; Bauer and Burton 1993), and “Proper Functioning Condition” (USDI Bureau of Land Management 1993).  In the context of this proposal, it is defined as the best channel condition, based on quantifiable morphological characteristics for each stream type.

We will apply a broad level geomorphic classification to the entire Hangman Creek watershed within the boundaries of the Reservation (Level I in Rosgen 1996).  Descriptive information on channel slope, channel shape, and channel patterns will be linked to the morphology of various valley types occurring on the Reservation.  Level I stream type classifications are based on geomorphic features than can be interpreted from aerial photography, topographic maps, geologic maps, soil maps, and a strong individual familiarity with the stream systems and landforms of the Hangman Creek watershed.  Field verification of the classifications will be completed for each of the major stream types and then incorporated into the Tribal GIS database using ArcInfo, Version 8.02.

A more detailed morphological description of stream types will be determined with field measurements from channel reaches identified as reference reaches (Level II and Level III in Rosgen 1996).  There will be at least one reference reach established for each Level I stream type that is identified.  Measured parameters will include bankfull width/depth ratio, entrenchment ratio, sinuosity, channel slope, channel materials, stream channel stability, and bank erosion hazard.  Channel materials will be quantified following the “pebble-count” method described by Wolman (1954).  Sample locations will be adjusted so that various bed features are sampled on a proportional basis along a given stream reach.  Stream channel stability will be evaluated using the system developed by Phankuch (1975).  This system has been widely used by the USFS and BLM.  Bank erosion hazard will be rated using the seven criteria described by Rosgen (1993).

This level of detail provides a mechanism to extrapolate site-specific data to stream reaches within the Hangman Creek watershed that have similar characteristics (Rosgen 1996).  More importantly, it will allow us to determine the departure of a target stream’s existing condition from a reference baseline prior to implementation.  These reference reaches will ultimately serve as templates for constructing stable channel configurations, implementing riparian enhancement projects and evaluating our progress in reaching the natural stream potential in the Hangman Creek watershed.

### Aquatic Habitat Inventories

Aquatic habitat inventories will be completed at reference reaches identified during the Level II and III channel surveys.  Habitat features will be measured according to the procedures described by Hankin and Reeves (1988).  Features that will be identified include channel units (pool, riffle, etc.) and measuring the length, mean wetted width, and mean and maximum depths.  They will also be measuring % cover; riparian vegetation and woody debris will also be noted.

#### Macroinvertebrate Bioassessment

The Coeur d'Alene Tribe Fisheries Program wants to assess the biological health of the Hangman Creek Watershed through the monitoring of aquatic macroinvertebrates.  Benthic samples will be collected from riffle/run habitats at three reference sites in the watershed (upper, middle, and lower) in order to capture existing variation in watershed condition.  Collection and analysis will follow EPA Rapid Bioassessment Protocol (RBP) II of Plafkin et al. (1989) and recent revisions (Chandler and Maret, 1991; Clark and Maret, 1991).  These initial samples will provide the biological baseline information, which will be compared to all additional samples, as well as information from reference sites.

Sample collection will take place at three times during the year to reflect seasonal changes in population characteristics (biomass, species richness, and number of individuals).  Two cross channel transects will be randomly selected (independent of prior sampling) at each of three sites (upper, middle and lower).  At each transect, a total of six quantitative benthic samples will be taken from the nearest riffle and run habitats using a modified 0.1m Hess bottom sampler (225um mesh, Waters and Knapp, 1961).  Samples will be composited for each transect.  We propose a field subsampling procedure that is consistent with the EPA RBP II.  The subsampling procedure will consist of evenly distributing the composite sample in a gridded pan with a light-colored bottom while in the field.  As grids are randomly selected, all organisms within those grids will be removed, until a minimum of 100 organisms have been selected from the sample.  All samples will be preserved onsite and delivered to a qualified laboratory for identification and analysis.

Macroinvertebrate data analysis will be performed according to the procedure outlined in the EPA RBP II.  A numerical value will be calculated for each of 8 metrics using raw numerical benthic macroinvertebrate data.  Calculated values can be compared to values derived from other sites in the region.  Each metric will be assigned a score according to the comparability (percent similarity) of calculated and reference data (Brusven, personal communication).  The percent comparison between the total scores provides a final evaluation of biologic condition.  The following eight metrics will be used: taxa richness; Hilsenhoff Biotic Index; ration of scrapers/scrapers + filtering collectors; ratio of EPT/chironomid + EPT; percent contribution of dominant taxon; IPT index; community similarity index; and ration of shredder/total.  Metrics will be calculated using the same macroinvertebrate sample.  Each sample will undergo taxonomic identification and enumeration, as well as a determination of functional feeding group.

Objective 6.	Monitor water quality as a means to determine if habitat enhancement efforts are effective.

Task 6a.	Collect baseline water quality data on the St. Joe River from mica creek down to the mouth excluding St. Maries River.

Task 6b.	Collect baseline water quality data on mainstem Coeur d'Alene River down to the mouth.

Task 6c.	Collect baseline water quality data on Coeur d'Alene Lake tributaries.

Methods:

Physical:

Each water quality site will be sampled using the hydrolab.  The hydrolab, a trade name for a multiparameter water quality testing probe, will be used to determine dissolved oxygen, pH, temperature, conductivity, and total dissolved solids from a calculation using conductivity measurements.  See methods for physical parameters described in Task 1b for a more detailed description.

A stage/discharge relationship based on a regression analysis-rating curve of staff height vs discharge will be completed.  Discharge measurements are taken at low, medium, and high flows in order to complete the rating curve.  Staff height is calculated from measurements taken from a staff gauge located in the stream.  Discharge measurements are taken in accordance to IFIM methodologies.  Using a measuring tape the stream is divided into 20 equal width cells.  Velocity of the water moving through the section is measured using a Gurley price AA digital pygmy flow meter.  Discharge for each cell is calculated by multiplying cell size X velocity in the cell.  All individual cell discharges are added together to determine total discharge (Q in cfs); Channel profiles are also taken in order to determine if the flow dynamics of the site are changing over time.

Selected streams have continuous temperature monitors installed.  RL 100 is a single channel, digital temperature monitor that is waterproof.  RL 100’s were set to record temperature every hour up to 75 days at which time they are downloaded and redeployed.

Water Collection:

Water samples will be collected using methods described in the Standard Methods for the Treatment of Water and Wastewater (APHA, 1995).  All analyses of this type will be conducted by an accredited (either state or EPA certification) contract laboratory (e.g. Spokane Tribal Laboratory).  Grab samples will be collected using a clean container fixed to a six foot pole or depth integrated samples will be taken using a DH-48 and immediately transferred to containers provided by the analytical laboratory.  All water samples are put on ice and are delivered to a contract laboratory the same day that they are collected.  All samples will follow strict chain of custody procedures as outlined in section 1060.B.: Chain of custody procedures (APHA, 1992).

Turbidity:

Turbidity is the amount of suspended inorganic and organic matter throughout the water column.  Turbidity measurements will be determined through use of a HACH Model 2100P portable turbidimeter.  The turbidimeter measures turbidity from 0.01 to 1000 Nephelometric Turbidity Units (NTU).  The unit will be calibrated according to manufacture specifications.  Frequency of calibration will be conducted as required for EPA reporting under NPDES standards.  As such, quarterly calibration of the unit using primary formazin standards will be completed.  Calibrations using Gelex secondary standards will be conducted on a bi-weekly basis.  Samples will be collected simultaneously with the hydrolab data.  The calibration results can be found in the master logbook.  In addition to using the HACH, depth integrated water samples will be sent to a contract laboratory to be analyzed as a means to check the accuracy of the instrument.  A contract laboratory using EPA method 180.1 will run duplicate samples for quality control.

Total Suspended Solids:

Depth integrated Total Suspended Solids (TSS) samples will be as described above in the water collection methods.  TSS is defined as the residue left on a filter paper of 2m or smaller after a portion of sample has been filtered through (APHA, 1992).  TSS is analyzed using EPA method 160.2: Gravimetric determination of Total Suspended Solids.

Nutrients:

Nutrient water samples will be taken as described in the water collection methods.  Nutrients will be analyzed using EPA method 300.0 Ion chromatography.  Ions sampled are chloride, fluoride, nitrate, nitrite, ortho-phosphate, and sulfate.

Total Kjeldahl Nitrogen:

Total Kjeldahl Nitrogen water samples will be taken as described in the water collection methods.  Organic nitrogen is defined functionally as organically bound nitrogen in the tri-negative oxidation state.  Analytically, organic nitrogen and ammonia can be determined together and have been referred to as “kjeldahl nitrogen, a term that reflects the technique used in their determination (Standard Methods 18th Edition, 1992).  Total kjeldahl nitrogen will be analyzed using standard method 4500-NorgB by a qualified laboratory.

Total Phosphate:

Total Phosphate water samples will be taken as described in the water collection methods.  Phosphorus is essential to the growth of organisms and can be the nutrient that limits the primary productivity.  Total phosphate measures the amount of orthophosphate, condensed phosphate and organically bound phosphate in the system.  Orthophosphates applied to agricultural or residential cultivated land as fertilizers are carried into surface waters with storm run off and to a lesser extent melting snow.  Small amounts of certain condensed phosphates are added to some water supplies during treatment and laundry detergents may also have condensed phosphates.  Organic phosphates are formed primarily by biological processes (Standard Methods 18th Edition, 1992).  Total phosphate will be analyzed using EPA method 200.7 Inductively Coupled Plasma by a qualified laboratory.

g. Facilities and equipment

The personnel for this Proposal will be housed in the current Coeur d'Alene Tribe Fish and Wildlife building and the newly constructed fish hatchery.  The proposal requires very little expenditure going to the purchase of new equipment.  The following is a list of equipment that we currently have and will be used, but is by no means comprehensive.

Fisheries Equipment:

- Custom 20 foot Electrofishing Boat 7.5 GPP Electrofisher

- Backpack Electroshocker Model 12-B Pow 400 Watt.

- Trimble Navigation GPS Pathfinder Basic Plus.

- Juniper Systems Pro 2000 and 4000 Field Computers.

- Gurley Model 1100 Digital Flow Meter.

- Minisonde Water Quality Multiprobe.

- Hydrolab Water Quality Multiparameter Water Quality Monitoring Instrument.

- HACH Model 2100P Trubidimeter.

- Ryan Temperature Monitors RL 100’s.

- GSA Vehicle

- Complete GIS map library

- Zooplankton, phytoplankton sampling nets, counting chambers and microscopes

- Pentium II Computer

h. References

| Reference (include web address if available online) | Submitted w/form (y/n) |
| --- | --- |
| Allen, D.B., K. Fite, J. Nelson, C. Medrow. 1998. Redband trout (Oncorhynchus mykiss gairdneri) population and stream habitat surveys in northern Owyhee County, Idaho and the Owyhee River and its tributaries. Idaho Bureau of Land Mangement, Technical Bulletin No. 98-14. | N |
| Allen, D.B., K. Fite, J. Nelson, B.J. Flatter. 1997a. Redband trout (Oncorhynchus mykiss gairdneri) population and habitat surveys in southern Owyhee County, Idaho. Idaho Bureau of Land Management, Technical Bulletin No. 97-10. | N |
| Allen, D.B., K. Fite, J. Nelson, B.J. Flatter. 1997b. Redband trout (Oncorhynchus mykiss gairdneri) population and habitat surveys in western Owyhee County, Idaho. Idaho Bureau of Land Management, Technical Bulletin No. 97-9. | N |
| Allen, D.B., B.J. Flatter, K. Fite. 1995. Redband trout (Oncorhynchus mykiss gairdneri) population and habitat surveys in Jump, Reynolds, and Sheep creeks, and sections of the Owyhee River in Owyhee County, Idaho. Idaho Bureau of Land Management, Technical Bulletin No. 95-6. | N |
| Allen, K.R. 1969. Limitations on production in salmonid populations in streams. In: T.G. Northcote (ed.) Symposium on salmon and trout in streams. University of British Columbia, Vancouver. Pp 3-18. | N |
| Allendorf, F. W. & Phelps, S. R. 1981. Use of allelic frequencies to describe population structure. Canadian Journal of Fisheries and Aquatic Sciences 38: 1507-1514. | N |
| American Public Health Association. 1992. Standard methods for the examination of water and wastewater. Victor Graphics, Inc. Baltimore Maryland. 18th Ed. | N |
| Armour, C.L., K.P. Burnham, and W.S. Platts. 1983. Field methods and statistical analyses for monitoring small salmonid streams. USDI, Fish and Wildlife Service. FWS/OBS-83/33. | N |
| Bauer, S.B. 1998. Lake Creek 1996 interim monitoring report, Kootenai County, Idaho. Kootenai-Shoshone Soil Conservation District. Coeur d’Alene, Idaho. 27p. | N |
| Bauer, S.B. and T.A. Burton. 1993. Monitoring protocols to evaluate water quality effects of grazing management on western rangeland streams. U.S. Environmental Protection Agency, 910/R-93-017, Washington, D.C. | N |
| Behnke, R.J. and R.L. Wallace. 1986. A systematic review of the cutthroat trout, Salmo clarki Richardson, a polytypic species. Pages 1-27 in J.S. Griffith, editor. The ecology and management of interior stocks of cutthroat trout. Special Publication of the Western Division, American Fisheries Society. | N |
| Beschta, R.L., R.E. Bilby, G.W. Brown, L.B. Holtby and T.D. Hofstra. 1987. Stream temperature and aquatic habitat: fisheries and forestry interactions. In Streamside Management: Forestry and Fishery Interactions, ed. E.O. Salo and T.W. Cundy, pp. 191-232. Institute of Forest Resources, University of Washington, Seattle, WA.. | N |
| Bisson, P.A. and eight coauthors. 1987. Large woody debris in forested streams in the Pacific Northwest: past, present, and future. Pages 143-190 in E.O. Salo and T.W. Cundy, editors. Streamside management: forestry and fishery interactions. University of Washington, Seattle. | N |
| Boll, J., D. van Buren, C.R. Campbell, E. Brooks, S. Chen, C. Stockle, D. McCool and D. Feichtinger. 2000. Distributed Modeling and Economic Analysis of Erosion in GIS for Watershed Restoration. To be presented at the Integrated Decision-Making for Watershed Restoration Symposium, Chevy Chase, Maryland, January 7-9, 2001. | N |
| Brusven, M.A. 1998. Department of Plant, Soil, and Entomological Sciences, University of Idaho, Moscow, ID, personal communication. | N |
| Chandler, G.L. and T.R. Maret. 1991. Protocols for assessment of biotic integrity (fish in Idaho streams. Water Quality Monitoring Protocols-Report 6. Idaho Dept. of Health and Welfare, Div. of Environ. Quality. Boise, Idaho. 7 pp. | N |
| Clark, W.H. and T.R. Maret. 1991. Protocols for assessment of biotic integrity (macroinvertebrates) in Idaho streams. Water quality Monitoring Protocols0Report #5. Idaho Dept. of Health and Welfare, Div. of Environ. Quality, Boise, Idaho. 7 pp. | N |
| Coeur d’Alene Tribe and others. 2000. Coeur d’Alene Subbasin Summary. Columbia Basin Fish and Wildlife Authority, Portland, Oregon. 94p. www.cbfwf.org | N |
| Coeur d'Alene Tribe Fish, Water, and Wildlife Program, 1998. Lake Creek Watershed Assessment. Environmental Protection Agency. 27pp | N |
| Conlin, K. and B.D.Tuty. 1979. Juvenile salmon field trapping manual. Dept. of Fisheries and Oceans. Fisheries and Marine Service Resource Services Branch, Habitat Protection Division, Vancoucer, B.C. 136p. | N |
| Ellis, M.N. 1932. Pollution of the Coeur d'Alene River and adjacent waters by mine wastes. U.S. Bureau of Fisheries. Mimeo Report. 55p. | N |
| Fajen, O.F. and R.E. Wehnes. 1981. Missouri’s method of evaluating stream habitat. In: Armantrout (ed.), Acquisition and Utilization of Aquatic Habitat Inventory Information. American Fisheries Society, Portland, Oregon. | N |
| Farr, I.S. and R.T. Clarke. 1984. Reliability of suspended load estimates in chalk stream. Arch. Hydrobiol. 102(1):1-19. | N |
| Fausch, K.D. 1984. Profitable stream positions for salmonids: relating specific growth rate to net energy gain. Canadian Journal of Zoology 62:441-451. | N |
| Ferguson, R.I. 1985. Accuracy and precision of methods for estimating river loads. Earth Surface Processes and Landforms 12:95-104. | N |
| Graves, S., K.L. Lillengreen, D.C. Johnson, and A.T. Scholz. 1992. Fisheries habitat evaluations on tributaries of the Coeur d’Alene Indian Reservation: Annual report 1990. U.S. Department of Energy, Bonneville Power Administration, Portland, Oregon. Project Number 90-044 | N |
| Hartman, G.F. 1965. The role of behavior in the ecology and interactions of underlying coho salmon (Oncorhynchus kisutch) and steelhead trout (Salmo gairdneri). Journal of the Fisheries Research Board of Canada 22:1035-1081. | N |
| Hankin, D.G., and G.H. Reeves. 1998. Estimating total fish abundance and total habitat area in small streams based on visual estimation methods. Can. J. Fish. Aquat. Sci. 45: 834-844. | N |
| Hickman, T. and R.F. Raleigh. 1982. Habitat suitability index models: Cutthroat trout. USDI, Fish and Wildlife Service. FWS/OBS-82/10.5. 38 pp. | N |
| Hodgson, T. 1998. Historical Tour of the Benewah Valley. Benewah Valley Association. Benewah, ID. | N |
| Huntingford, F.A., N.B. Metcalfe, and J.E. Thorpe. 1988. Choice of feeding station in Atlantic salmon, Salmo salar, parr: effects of predation risk, season and life history strategy. Journal of Fish Biology 33:187-211. | N |
| IDFG. 1996. Fisheries Management Plan, 1996-2000. Idaho Department of Fish and Game, Boise, ID. | N |
| Jearld, T. 1983. Age determination. in: Nielsen, L.A. and D.L. Johnson (eds.), Fisheries Techniques. American Fisheries Society, Bethesda, MD. 468p. | N |
| Kappesser, G. 1992. Riffle Armor Stability Index. Version 4.1. USDA, Forest Service. 7p. | N |
| Keller, E.A. and F.J. Swanson. 1979. Effects of large organic material on channel form and fluvial processes. Earth Surface Processes 4:361-380. | N |
| Kellerhals, R., M. Church, and D.I. Bray. 1976. Classification and analysis or river processes. J. Hydraul. Div., ASCE 102(HY7). 813-829. | N |
| Kellerhals, R., C.R. Neill and D.I. Bray. 1972. Hydraulic and geomorphic characteristics of rivers in Alberta. Research Council of Alberta, River Engineering and Surface Hydrology Report 72-1. 52pp | N |
| Knudsen, K.L. and P. Spruell. 1999. Genetic analysis of westslope cutthroat trout in tributaries of Coeur d’Alene Lake. Coeur d’Alene Tribe Fisheries Program, Final Report WTSGL99-106, Plummer, Idaho. | N |
| Kootenai-Shoshone Soil Conservation District. 1991. Agricultural pollution abatement plan, Lake Creek watershed. Final Planning Report. USDA Soil Conservation Service, Coeur d’Alene, Idaho. | N |
| Kunkle, S.H. and G.H. Comer. 1971. Estimating suspended sediment concentration in streams by turbidity measurements. J. Soil and Water Conservation. Jan-Feb: pages 18-20. | N |
| Leopold, L.B., M.G. Wolman and J.P. Miller. 1964. Fluvial processes in geomorphology. Freeman, San Francisco, CA. 522pp. | N |
| Lillengreen, K.L., A.J. Vitale, R. Peters. 1999. Coeur d’ Alene Tribe Management Plan- enhancement of residentfish resources within the Coeur d’ Alene Indian reservation. U.S. Department of Energy, Bonneville Power Administration. Project Number 90-044 | N |
| Lillengreen, K., A.J. Vitale, R. Peters. 1996. Fisheries habitat evaluation on tributaries of the Coeur d’ Alene Indian Reservation: 1993, 1994 Annual Report. U.S. Department of Energy, Bonneville Power Administration, Portland, Oregon. Project Number 90-044. www.efw.bpa.gov | N |
| Lillengreen, K.L., A.J. Vitale, R. Peters. 1998. Coeur d’Alene Tribe project management plan: enhancement of resident fish resources within the Coeur d’Alene Indian Reservation. Bonneville Power Administration, Project Number 90-044, Portland, Oregon. | N |
| Lillengreen, K.L., D.C. Johnson, and A.T. Scholz. 1993. Fisheries habitat evaluations on tributaries of the Coeur d’Alene Indian Reservation: Annual report 1991. U.S. Department of Energy, Bonneville Power Administration, Portland, Oregon. Project Number 90-044 | N |
| Lillengreen, K.L., T. Skillingstad, and A.T. Scholz. 1994. Fisheries habitat evaluations on tributaries of the Coeur d’Alene Indian Reservation: Annual report 1992. U.S. Department of Energy, Bonneville Power Administration, Portland, Oregon. Project Number 90-044 | N |
| Mallet, J. 1969. The Coeur d’ Alene Lake fishery. Idaho Wildlife Review. May-June, 1969. 3-6 p. | N |
| Mallet, J. 1968. Coeur d’Alene Lake fisheries investigations. Idaho Dept. of Fish and Game. 3-6 pp. | N |
| Malvestuto, S. P. 1983. Sampling the recreational fishery. In: Nielsen, L.A. and D.L. Johnson (eds.), Fisheries Techniques. American Fisheries Society, Bethesda, Maryland. P 397-419 | N |
| Mollard, J.D. 1973. Air photo interpretation of fluvial features. In: Fluvial Processes and Sedimentation. Research Council of Canada. 341-380. | N |
| Montgomery, D.R. and J.M. Buffington. 1993. Channel classification, prediction of channel response, and assessment of channel condition. Wash. Dpt. Nat. Res. Rep. TFW-SH10-93-002. Olympia, WA. 86p. | N |
| Oien, W.E. 1957. A pre-logging inventory of four trout streams in northern Idaho. M.S. Thesis. University of Idaho. Moscow, Idaho. 92p. | N |
| Peters, R., A.J. Vitale, and K.L. Lillengreen. 1999. Supplementation Feasibility Report on the Coeur d'Alene Indian Reservation. U.S. Department of Energy, Bonneville Power Administration. Project Number 90-044. www.efw.bpa.gov | N |
| Peters, R., K.L. Lillengreen and A.J. Vitale. 2000. Coeur d’Alene Tribe Trout Production Master Plan. U.S. Department of Energy, Bonneville Power Administration, Portland, Oregon. Project Number 90-044. www.efw.bpa.gov | N |
| Peters, R.L. and A.J. Vitale. 1999. Stock Assessment of westslope cutthroat trout on the Coeur d’Alene Reservation. Technical Report. Coeur d’Alene Tribe, Fisheries Program, Plummer, Idaho. | N |
| Pfancuch, D.J. 1975. Stream reach inventory and channel stability evaluation. USDA, Forest Service. 15p. | N |
| Plafkin, J.L., M. Barbour, K. Porter, S. Gross, and R. Hughes. 1989. Rapid bioassessment protocols for use in streams and rivers: benthic macroinvertebrates and fish. EPA Report 444/4-89-001, Washington, D.C. | N |
| Platts, W.S. 1990. Managing fisheries and wildlife on rangelands grazed by livestock. Nevada Department of Fish and Wildlife. | N |
| Platts, W.S. and R.L. Nelson. 1989. Stream canopy and its relationship to salmonid biomass in the Intermountain west. North American Journal of Fisheries Management 9:446-457. | N |
| Platts, W.S., and 12 coauthors. 1987. Methods for evaluating riparian habitats with applications to management. USDA Forest Service, General Technical Report, INT-221. 177 pp. | N |
| Rieman, B.E. and J.D. McIntyre. 1993. Demographic and habitat requirements for conservation of bull trout. USDA Forest Service, General Technical Report INT-302, Ogden, UT. | N |
| Reiman, B.E. and K.A. Apperson. 1989. Status and analysis of salmonid fisheries: Westslope cutthroat trout synopsis and analysis of fishery information. Idaho Dept. of Fish and Game, Job Performance Report, Project F-73-R-11. Boise, Idaho. | N |
| Reynolds, J.B. 1983. Electrofishing. in: Nielsen, L.A. and D.L. Johnson (eds.), Fisheries Techniques. American Fisheries Society, Bethesda, MD. 468p. | N |
| Rosgen, D.L. 1994. Classification of natural rivers. Catena 22:169-199. | N |
| Rosgen, D.L. 1996. Applied river morphology. Wildland Hydrology, Pagosa Springs, CO. 246 pp. | N |
| Rosgen, D.L. 1993. Applied fluvial geomorphology, training manual. River Short Course, Wildland Hydrology, Pagosa Springs, Co. 450pp | N |
| Scholz, A.T., D.R. Geist, and J.K. Uehara. 1985. Feasibility report on restoration of Coeur d’Alene Tribal Fisheries. Upper Columbia United Tribes Fisheries Center. Cheney, WA. 85 p. | N |
| Scholz, A. T., and 9 coauthors. 1985. Compilation of information on salmon and steelhead total run size, catch and hydropower related losses in the upper Columbia River Basin, above Grand Coulee Dam. Fisheries Technical Report No. 2, UCUT. Cheney, WA | N |
| Scott, W.B. and E.J. Crossman. 1973. Freshwater fishes of Canada. J. Fish. Res. Bd. Can., Bulletin 184. | N |
| Seber, G.A.F., and E.D. LeCren. 1967. Estimating population parameters from catches large relative to the population. Journal Animal Ecology 36:631-643. | N |
| Shirvell, C.S. 1990. Role of instream rootwads as juvenile coho salmon (Oncorhynchus kisutch) and steelhead trout (O. mykiss) cover habitat under varying streamflows. Canadian Journal of Fisheries and Aquatic Sciences 47:852-861. | N |
| Soil Conservation Service. 1975. Computing sheet and rill erosion using the Universal Soils Loss Equation (USLE). Agronomy Technical Note #19. USDA Soil Conservation Service, Boise, Idaho. | N |
| Soule, M.E., editor. 1987. Viable populations for conservation. Cambridge University Press, Cambridge, England. | N |
| Spokane County Water Conservation District. 1994. Washington State. | N |
| Spruell, P., K.L. Knudsen, J. Miller, and F.W. Allendorf. 1999. Genetic analysis of westslope cutthroat trout in tributaries of Coeur d’Alene Lake. Coeur d’Alene Tribe Fisheries Program, Progress Report WTSGL99-101, Plummer, Idaho. | N |
| State of Idaho. 1996. Governor Philip E. Batt’s Bull Trout Conservation Plan. | N |
| St. Maries Gazette Record. 1911-1918, St Maries, Idaho. | N |
| Thomas, R.B. 1985. Estimating total suspended sediment yield with probability sampling. Water Resources Research 21(9):1381-1388. | N |
| USDA Forest Service, USDI Bureau of Land Management. 1996. Status of the Interior Columbia Basin, Summary of Scientific Findings. USDA, Forest Service, General Technical Report PNW-GTR-385. Portland, OR. | N |
| USEPA. 1996. National Water Quality Inventory, 1996 Report to Congress. EPA 841-R-97-008. Washington, DC: Office of Water. | N |
| U.S. Department of Agriculture, USFS/BLM. 1997. Interior Columbia Ecosystem Management Project. Draft environmental impact statement. | N |
| U.S. Department of Agriculture. 1998. Biological Assessment St. Joe and North Fork Clearwater Rivers. US Forest Service. | N |
| USFWS. 1998. Determination status for the Klamath River and Columbia River Distinct Population Segments of Bull Trout. USDI, Fish and Wildlife Service. Federal Register, 63(111). Pg. 31647-31674. | N |
| USDI, Bureau of Land Management. 1993. Riparian area management Tech. Ref. 1737-9. BLM Service Center, Denver, Colorado. | N |
| USDA Forest Service. 1992. Integrated riparian evaluation guide. USDA Forest Service, Intermountain Region, Ogden, UT. | N |
| Van Deventer, J.S., and W.S., Platts. 1989. Microcomputer software system for generating population statistics from electrofishing data-user’ guide for MicroFish 3.0. Gen. Tech. Rep. INT-254. Ogden, UT: U.S. Department of Agriculture, Forest Service, Intermountain Research Station. 29 p. | N |
| VanSickle, J. and R.L. Beschta. 1983. Supply-based models of suspended sediment transport in streams. Water Resources Research 19(3):768-778. | N |
| Vitale, A.J. 2000. Status and recovery potential for westslope cutthroat trout (Oncorhynchus clarki lewisi) in waters of the Coeur d’Alene Indian Reservation. Presented at American Fisheries Society Annual Meeting, Coeur d’Alene, Idaho. March. | N |
| Vitale, A.J. 1999. Stream restoration approaches on the Coeur d’Alene Reservation. Presented at The Value of Native Coldwater Fish, Coeur d’Alene, ID. February. | N |
| Vitale, A.J., D.A. Bailey, and R. Peters and K.L. Lillengreen. 1999. Implementation of fisheries enhancement opportunities on the Coeur d’Alene Indian Reservation. 1998 Annual Report to the Bonneville Power Administration. U.S. Department of Energy, Bonneville Power Administration, Portland, Oregon. Project Number 90-044. | N |
| Vitale, A.J. 1997. Restoration planning and implementation: Using watershed analysis to identify effective restoration tools. Presented at Landscape Connections: Restoring Ecological Integrity in the Inland Northwest, Washington State University, Pullman, WA. September. | N |
| Walling, D.E. 1977. Assessing the accuracy of suspended sediment rating curves for a small basin. Water Resources Research 13(3):531-538. | N |
| Washington Forest Practices Board. 1997. Standard methodology for conducting watershed analysis. Version 4.0. Olympia, WA. | N |
| Washington Forest Practices Board. 1992. Standard methodology for conducting watershed analysis. Version 1.1. Olympia, WA. | N |
| Waters, T.F., and R.J. Knapp. 1961. An improved bottom sampler. Trans. Amer. Fish. Soc. 90:225-226. | N |
| Wolman, M.G. 1954. A method of sampling coarse river-bed material. Transactions of the American Geophysical Union. 35(6):951-956. | N |
| Zippen, C. 1958. The removal method of population estimation. Journal of Wildlife Management 22:82-90 | N |

# Section 10 of 10. Key personnel

Summary of Key Personnel

| Name | Title | FTE/Hours | Experience |
| --- | --- | --- | --- |
| Ron Peters | Program Manager | Part time | 12 years |
| Jeff Jordan | Hatchery Supervisor | Full time | 11 years |
| Dee Ann Bailey | Research Supervisor | Full time | 6 years |
| TBD | Project Biologist | Full time | Unknown |
|  | Technicians (4-5) | Seasonal |  |

Resumes

### Ron Peters

Education and Certifications:

B.S.; Eastern Washington University; Zoology/Fisheries Management; 1987

M.S.: Eastern Washington University; Fish Pathology/Fisheries Management;1995

Current Employer and Responsibilities:

Coeur d’Alene Tribe; Fisheries Program Manager; 2/2000 to Present

Responsible for oversight, coordination and implementation of all fisheries projects undertaken by the Coeur d’Alene Tribe. Principle responsibilities include supervision of professional and technical staff, preparation of policy recommendation for Council Action, preparation and approval of implementation and monitoring plans, annual reports, and budgets.

Previous Employment:

Coeur d’Alene Tribe; Fisheries Biologist; 5/1996 to 2/2000

Quinault Indian Nation; Fisheries Biologist; 1992 to 1996

Upper Columbia United Tribes; Research Associate; 1991 to 1992

Eastern Washington University; Research Assistant; 1989 to 1991

Expertise:

Mr. Peters has over ten years professional experience in the evaluation and management of aquatic ecosystems.  In other positions, he coordinated and implemented BPA funded harvest enhancement projects. Was responsible for all aspects of Coeur d'Alene Tribe water quality monitoring and analysis.  Supervised professional and technical staff, prepared and implemented monitoring and evaluation plans.  Prepared of quarterly and annual reports, and budgets.  He was also the lead technical person in charge of management of the Quinault River sockeye salmon run.  His duties included collecting, recording, and interpreting information related to the enhancement and preservation of the Quinault River sockeye salmon run.  He was also lead investigator in charge of the Quinault Indian Nation Water Quality Laboratory where primary duties included oversight of all activities, development of experimental design, quality control, and data analysis.

Publications and Presentations:

Peters, R.L.., K.L. Lillengreen and A.J. Vitale.  2000.  Coeur d’Alene Tribe Trout Production Master Plan. U.S. Department of Energy, Bonneville Power Administration, Portland, Oregon.  Project Number 90-044.

Peters, R.L.., A.J. Vitale, and K.L. Lillengreen.  1999.  Supplementation Feasibility Report on the Coeur d'Alene Indian Reservation.  U.S. Department of Energy, Bonneville Power Administration.  Project Number 90-044.

Vitale, A.J., D.A. Bailey, and R.L Peters and K.L. Lillengreen.  1999.  Implementation of fisheries enhancement opportunities on the Coeur d’Alene Indian Reservation.  1998 Annual Report to the Bonneville Power Administration.  U.S. Department of Energy, Bonneville Power Administration, Portland, Oregon.  Project Number 90-044.

Peters, R.L. and A.J. Vitale. 1999.  Stock Assessment of westslope cutthroat trout on the Coeur d’Alene Reservation.  Technical Report.  Coeur d’Alene Tribe, Fisheries Program, Plummer, Idaho.

Lillengreen, K., A.J. Vitale, R.L., Peters.  1999.  Coeur d’Alene Tribe project management plan - enhancement of resident fish resources within the Coeur d’Alene Indian Reservation.  U.S. Department of Energy, Bonneville Power Administration.  Project Number 90-044.

Lillengreen, K.L, A.J. Vitale, R.L. Peters.  1996.  Fisheries habitat evaluation on tributaries of the Coeur d’ Alene Indian Reservation: 1993, 1994 Annual Report.  U.S. Department of Energy, Bonneville Power Administration, Portland, Oregon.  Project Number 90-044.

Peters, R.L. 1995.  Ecological investigations into the life history of the nematode Eustrongylides sp. (Nematoda: Dioctophymatoidea) found in Franklin D. Roosevelt Lake, WA.  M.S. Thesis.   Eastern Washington University.  Cheney, WA.  pp. 83.

Peters, R.L.  1994.  Hydroacoustic estimate of escapement of Quinault River sockeye salmon.  Presented to North Pacific International Chapter American Fisheries Society.  March.

Jeffery Jordan

Education:

BS; Eastern Washington University: 1998

Completed Cold Water Fish Culture  USFWS, 1998

Complete Fish Disease and Pathology USFWS, 1999

Completed One Full season as Hatchery intern at the IDFG facility in Clark Fork, ID

Current Employer and Responsibilities

Coeur d’ Alene Tribe of Indians; Tribal Biologist;  August 1998- Present

Complete tasks related to Coeur d'Alene Lake shoreline habitat study

Complete tasks related to construction of Coeur d'Alene Tribe Trout Production

Facility. Supervision of technical staff

Preparation of annual workplans, annual reports, and budgets.

Previous Employment

Coeur d’ Alene Tribe of Indians; Seasonal Technician; 1994- 1997

Upper Columbia United Tribes Fishier Research Center; Seasonal Technician; 1994-1998

Expertise:

Mr. Jordan just completed his bachelors degree requirements two years ago.  He has worked on several fisheries related projects for the Coeur d'Alene Tribe Fish, Water, and Wildlife Program.  He has worked on several stream restoration projects, as well as, lake studies projects, and water quality monitoring projects.  His work at the University included rainbow trout and kokanee salmon tagging, migration, and imprinting studies on Lake Roosevelt, WA.,

Dee Ann Bailey

Education and Certifications:

B.S.; University of Idaho; Fisheries Management; 1995

Aerial photo interpretation of renewable natural resources workshop.  Moscow, ID., 1999.

Current Employer and Responsibilities:

Coeur d’Alene Tribe; Fisheries Biologist; 10/98 to Present

Responsible for coordination and implementation of all research, monitoring and evaluation studies within the fisheries program. Also, responsible for quality assurance/quality control of water quality data collection and analysis on the Reservation.  Other responsibilities include the planning and construction of “put and take” fishing ponds that are integral to the Compensatory Harvest Project.  Mrs. Bailey supervises technician staff, and prepares annual work plans, budgets and reports.

Previous Employment:

Coeur d’Alene Tribe; Water Resources Technician; 5/97 to 10/98

Pacific Northwest National Laboratory (PNNL), Battelle; 1996

US Forest Service, Walla Walla Ranger District; 1995

Expertise:

Mrs. Bailey has five years professional experience in the evaluation of fisheries habitat and populations.  She has assisted in the collection and analysis of habitat and population data in both riverine and lacustrine environments.  In other positions, she mapped fisheries habitat characteristics in the Hanford Reach of the Columbia River and assisted in the evaluation of spawning conditions for chinook salmon.

Publications and Presentations:

Vitale, A.J., D.A. Bailey, and R. Peters and K.L. Lillengreen.  1999.  Implementation of fisheries enhancement opportunities on the Coeur d’Alene Indian Reservation.  1998 Annual Report to the Bonneville Power Administration.  U.S. Department of Energy, Bonneville Power Administration, Portland, Oregon.  Project Number 90-044.

Policies and measures for resident fish substitution are in Sections 10.1 and 10.2 of the Council’s Fish and Wildlife Program.  The intent of this policy is to replace losses of anadromous fish in areas now permanently blocked to salmon and steelhead with resident fish species.